# SlyLED User Manual — 3D Volumetric Lighting System (v1.0)

Licensing. SlyLED is distributed under the PolyForm Noncommercial 1.0.0 licence — free for personal, hobbyist, educational, research, and nonprofit use. Paid client engagements, corporate internal use, embedded products, and paid services built on SlyLED require a separate commercial licence. See <https://electricrv.ca/slyled/licensing> or write to <info@electricrv.ca>.

## Table of Contents

Getting Started

Walkthrough: First Show in 30 Minutes

Platform Guide

Fixture Setup

Stage Layout

Stage Objects

Creating Spatial Effects

Track Action

Building a Timeline

Baking & Playback

Show Preview Emulator

DMX Fixture Profiles

Preset Shows

Camera Nodes

Firmware & OTA Updates

System Limits

Troubleshooting

Examples

API Quick Reference

Glossary

Appendix A — Camera Calibration Pipeline

Appendix B — Moving-Head Calibration Pipeline

Appendix C — Documentation Maintenance

Appendix D — Idle Behaviour

Appendix E — Remote Control: Android Phone & Gyro Gyro

────────────────────────────────────────

## 1. Getting Started

SlyLED is a three-tier LED and DMX lighting control system:

Orchestrator (Windows/Mac desktop app or Android app) — design shows and control playback

Performers (ESP32/D1 Mini) — run LED effects on hardware

DMX Bridge (Giga R1 WiFi) — output Art-Net/sACN to DMX fixtures

### Quick Start

Launch the desktop app: powershell -File desktop\windows\run.ps1 (Windows) or bash desktop/mac/run.sh (Mac)

Open the browser at http://localhost:8080

Go to Setup tab, click Discover to find performers on your network

Go to Layout tab to position fixtures on the stage

Go to Runtime tab, load a Preset Show, click Bake & Start

────────────────────────────────────────

## 2. Walkthrough: First Show in 30 Minutes

This walkthrough builds a complete DMX moving-head show from scratch — hardware discovery, fixture setup, layout, camera registration, actions, timeline, and playback. Every step was validated end-to-end during QA testing (issue #533). Follow in order; each step builds on the last.

What you need:

SlyLED orchestrator running on Windows or Mac

At least one DMX moving head connected via Art-Net/sACN bridge (e.g. Enttec ODE Mk3)

At least one USB camera node on the network (Orange Pi or Raspberry Pi)

All devices on the same LAN subnet as the orchestrator

────────────────────────────────────────

### Step 1 — Launch and Create a New Project

Start the orchestrator:

powershell -File desktop\windows\run.ps1

Open http://localhost:8080 in Chrome or Edge. The SPA loads on the Dashboard tab.

SPA at launch showing Dashboard tab

Go to Settings tab → Project → click New Project, then name it (e.g. "Walkthrough Show").

New project dialog

────────────────────────────────────────

### Step 2 — Set Stage Dimensions

In Settings → Stage, enter the dimensions of your performance area:

Width: 6000 mm (6 m)

Height: 3000 mm (3 m)

Depth: 4000 mm (4 m)

Click Save. The layout canvas will resize to match these dimensions.

────────────────────────────────────────

### Step 3a — Discover DMX Hardware

Go to the Setup tab. In the DMX Nodes section, click Discover Nodes. SlyLED broadcasts an ArtPoll packet; Art-Net bridges on the network reply within 3 seconds.

Setup tab after hardware discovery — Art-Net node shown

Any discovered nodes appear in the list with their IP, port, and universe count. If your bridge is not found:

Confirm it is powered and on the same LAN subnet

Check that UDP port 6454 is not blocked by a local firewall

Some bridges require the Art-Net source IP to match their configured subnet

────────────────────────────────────────

### Step 3b — Configure and Start the DMX Engine

Go to Settings → DMX:

Universe Routing: Set Universe 1 → your Art-Net node IP (or leave as broadcast 255.255.255.255 to reach all nodes on the subnet).

Click Start Engine. The status indicator turns green ("Running").

DMX engine configuration — universe routing and start

DMX routing — universe 1 assigned to bridge

Important: The engine must be running before adding DMX fixtures or running calibration. If you stop and restart the orchestrator, re-start the engine here.

────────────────────────────────────────

### Step 4 — Add DMX Moving Head Fixtures

Go to the Setup tab → click + DMX Fixture. The fixture wizard opens.

Finding the right profile:

In the Search box, type your fixture's name (e.g. "Sly Moving Head Super Mini")

Results show Local profiles first, then Community (from the shared library), then OFL (Open Fixture Library)

If a community profile download fails ("imported: 0"), it may contain unsupported channel types — fall back to a local generic profile or search OFL directly

For a generic 16-channel moving head with no exact match, search OFL for "moving head" and import the closest match

Fixture 1 (stage left):

Name: MH1 SL

Universe: 1, Start Address: 1

Profile: your moving head profile

Click Create Fixture

Moving head 1 added to Setup tab

Fixture 2 (stage right):

Name: MH2 SR

Universe: 1, Start Address: 17

Profile: same profile

Click Create Fixture

Moving head 2 added to Setup tab

────────────────────────────────────────

### Step 5 — Add a Wash or Spot Fixture

Add any additional fixtures (e.g. a 350W wash spot):

Name: Spot C

Universe: 1, Start Address: 33

Profile: your spot/wash profile

350W spot added to Setup tab

────────────────────────────────────────

### Step 6 — Register Camera Nodes

Still on the Setup tab, scroll to the Camera Nodes section. Click Discover Cameras. Camera nodes running slyled-cam respond to the same UDP broadcast as performers.

Alternatively, enter the camera's IP manually and click Add.

Camera 1 (left):

IP: your camera's IP (e.g. 192.168.10.50)

Name: Cam Left

Camera 2 (right):

IP: your second camera's IP

Name: Cam Right

Two cameras added to Setup tab

Each camera appears with online/offline status. Click Snap to verify the live feed.

[Image: screenshots/walkthrough-533/06-cam1_left_hires.png — not found]

[Image: screenshots/walkthrough-533/06-cam2_right.png — not found]

Note: Camera discover sometimes returns 0 nodes on the first broadcast due to UDP timing. If no cameras are found, wait 3 seconds and click Discover again. This is a known intermittency (#542) being addressed in a future release.

────────────────────────────────────────

### Step 7 — Position All Fixtures on the Layout

Switch to the Layout tab. All added fixtures appear in the left sidebar as "unplaced."

Place and position each fixture:

Click a fixture in the sidebar to select it

Click on the canvas to place it, or drag from the sidebar

Double-click the placed fixture to open the edit dialog and enter exact coordinates

| Fixture | X (mm) | Y (mm) | Z (mm) |
| --- | --- | --- | --- |
| MH1 SL | 1500 | 3000 | 500 |
| MH2 SR | 4500 | 3000 | 500 |
| Spot C | 3000 | 3000 | 500 |
| Cam Left | 0 | 2500 | 0 |
| Cam Right | 6000 | 2500 | 0 |

Click Save after entering coordinates for each fixture.

Layout tab — fixtures placed at initial positions

[Image: screenshots/walkthrough-533/04d-layout-positions.png — not found]

[Image: screenshots/walkthrough-533/06c-cameras-positioned.png — not found]

Tip: Use 3D view (toggle in the layout toolbar) to visually verify that movers are elevated on the truss and aimed downward toward the stage floor.

────────────────────────────────────────

### Step 8 — Add a Stage Object

Go to the Layout tab → click + Object in the toolbar.

Name: Music Stand

Type: Prop (moving — trackable by movers)

Position: X: 3000, Y: 0, Z: 2000 (center stage, at floor level, mid-depth)

Size: 300 × 1200 × 300 mm

Click Save. The object appears as a labeled rectangle on the canvas.

Music stand object on layout

────────────────────────────────────────

### Step 9 — Run Mover Calibration

Before the moving heads can accurately track positions, calibrate each one. This step requires the camera nodes to be positioned on the layout (Step 7) and the DMX engine running (Step 3b).

In the Layout tab, double-click MH1 SL. Click Calibrate.

Calibration buttons in fixture edit dialog

[Image: screenshots/walkthrough-533/07-calibrate-ui.png — not found]

Select Green as the beam color (good contrast on dark floors)

Click Start Calibration

The wizard runs automatically through eight phases: warmup → discovery → blink-confirm → mapping/convergence → grid build → verification sweep → model fit → held-out parametric gate → save

Repeat for MH2 SR

Calibration typically takes 2–4 minutes per head. For the complete phase-by-phase reference — what each phase does, how long it should take, what fallbacks exist, and what to check when a phase stalls — see Appendix B — Moving-Head Calibration Pipeline.

────────────────────────────────────────

### Step 10 — Create Actions

Go to the Actions tab. You'll create two actions: a static aim and a figure-eight sweep.

Action 1: Aim Red (static spotlight)

Click + New Action

Name: Aim Red

Type: DMX Scene

Colour: Red (255, 0, 0)

Dimmer: 255

Click Save Action

Aim Red action — aimed at stage center

Action 2: Figure Eight (dynamic sweep)

Click + New Action

Name: Figure Eight

Type: Track

Target Objects: leave empty (track all moving objects)

Cycle Time: 4000 ms

Click Save Action

Figure Eight track action

────────────────────────────────────────

### Step 11 — Build a Timeline

Go to the Runtime tab (labeled Shows in some versions). Click + New Timeline.

A dialog prompts for the name — enter Walkthrough Show. A second dialog prompts for duration — enter 120 (seconds). Click OK.

Timeline editor with tracks

Add tracks:

For each fixture or group, click + Add Track:

Track for MH1 SL — add clip: Aim Red at 0s, duration 10s

Track for MH1 SL — add clip: Figure Eight at 10s, duration 110s

Track for MH2 SR — add clip: Figure Eight at 0s, duration 120s

Track for All Performers — add clip with ambient wash color

The Track action (type 18) evaluates in real-time during playback and doesn't need to be baked per-frame — it reads live object positions at 40 Hz.

────────────────────────────────────────

### Step 12 — Bake and Start Playback

Click Bake — the engine compiles the timeline into per-fixture action sequences. Progress shows frame count.

Click Start — NTP-synchronized playback begins.

Watch the Runtime view:

Beam cones animate in 3D as the timeline plays

The figure-eight pattern moves through the stage space

DMX output is sent via Art-Net to the physical fixtures

Runtime view with animated beam cones

To test a blackout:

Click Stop, then fire a Blackout action from the Settings → Group Control panel

Blackout state — all beams off

────────────────────────────────────────

### Step 13 — Save the Project

Go to Settings → Project → click Export. A .slyshow file is downloaded containing all fixtures, layout positions, objects, camera registrations, calibration data, actions, and timelines.

To reload: Settings → Project → Import → select the .slyshow file.

Project saved — all state bundled in .slyshow file

────────────────────────────────────────

### Walkthrough Troubleshooting

| Problem | Solution |
| --- | --- |
| No Art-Net nodes discovered | Confirm bridge is on the same subnet; UDP port 6454 not blocked |
| DMX engine won't start | Check Settings → DMX → verify universe routing is configured |
| Community profile download fails | Profile has unsupported channel types — use local or OFL profile instead |
| Fixture position resets to 0,0,0 | Ensure saveFixture() completes before switching tabs; use the edit dialog Save button |
| Camera discover returns 0 | Wait 3s and retry — first broadcast may arrive before socket is bound (#542) |
| Calibration fails to detect beam | Dim ambient light, verify beam color contrasts with floor, check camera can see the beam |
| Figure Eight doesn't move heads | Verify Track action has no trackFixtureIds restriction; confirm engine is running |
| Timeline tracks missing after create | Add tracks manually after timeline creation — they are not auto-created |

────────────────────────────────────────

## 3. Platform Guide

### Windows Desktop (SPA)

The primary design and control interface. Full-featured 7-tab SPA with 2D/3D layout, timeline editor, spatial effects, DMX profiles, and firmware management.

Launch: powershell -File desktop\windows\run.ps1 or run SlyLED.exe Install: Run SlyLED-Setup.exe (includes system tray icon)

### Android App

Live operator tool for running shows from your phone. Connects to the desktop server over WiFi. As of v1.8.1 the Control tab is rebuilt as a Command Surface — see #888 / docs/design/mobile_ui_redesign.md.

Install: Sideload slyled-android.apk to your phone and install. Connect: Scan the QR code on the desktop Settings tab, or enter the server IP and port manually.

Android Connect screen

Bottom nav (3 tabs): Stage / Control / Status. Settings lives at the top-right ⚙ gear, not in the bottom nav.

Top bar gestures:

Long-press the SlyLED logo → instant blackout (master = 0). Heavy double-haptic. The only "nuclear" gesture; other safety actions live as per-page buttons.

Connection pill — green dot = Connected; orange slow pulse = Reconnecting (Degraded); red fast pulse = Offline. Tap to retry.

⚙ Settings gear — server name, stage dimensions, Auto Brightness calibration, config export/import, disconnect.

Stage tab — live viewport showing all fixtures with beam cones, tracked object markers, grid floor. Pinch-to-zoom + drag-to-pan.

Android Stage view

Control tab (rebuilt for v1.8.1): persistent Now Playing anchor over a 4-page pager.

Master (default page) — global brightness slider with ±5% steppers + bloom-on-drag. Auto Brightness toggle (moved from Settings) + source picker (Mic / Playback / USB) + live envelope meter.

Control · Master

Grab — moving-head tiles showing current colour + pan/tilt direction arrow. Favourites row at top (star to add). Tap a tile → Controller Mode (gyro-driven pan/tilt at 20 Hz). Top-right "Send all home" button homes every mover.

Control · Grab

Fixtures — non-mover DMX fixtures (bubble machines, hazers, washes, pars, strobes) with profile-driven shortcuts: 🫧 bubbles, 💨 haze low/med/high, 🌀 fan slow/med/fast, 💡 colour swatches, 🟣 UV, ⚡ momentary strobe, 🧼 hold-to-clean. "More controls →" opens a per-channel sheet with capability sliders. Top-right "Stop all effects" kills strobes + bubble/haze in parallel.

Control · Fixtures

Shows — starred → recent → all sections, ranked by last-played time. One-tap launch. Long-press to star.

Control · Shows

Now Playing anchor sits above the pager — name, loop chip, elapsed / total, progress bar, STOP and Next.

Status tab — device monitoring (performers online/offline, RSSI, firmware), camera nodes with Track button to start/stop person tracking, and Art-Net/DMX engine status.

Android Status

Settings sheet (top-right ⚙) — System name, units, stage dimensions (W × H × D), dark mode, logging, plus the Auto Brightness configuration block (enable, model mode, sensitivity/floor/ceiling/attack/release sliders).

Android Settings

Controller Mode (Grab → tap a mover): Hold the phone and point where you want the beam — pan/tilt follows your phone orientation at 20 Hz. Tap Recenter to calibrate, X to exit. Press-Start / press-Stop guarded by nonce+ACK (#825). The first time you use it on a new phone, the aim-axis wizard (#869) measures your phone's body-frame axes; takes ~10 seconds.

### Firmware Config (ESP32/D1 Mini)

Each performer serves a 3-tab config page at http://<device-ip>/config:

Dashboard — hostname, firmware version, active action status

Settings — device name, description, string count

Config — per-string LED count, length, direction, GPIO pin (ESP32)

────────────────────────────────────────

## 4. Fixture Setup

### What Are Fixtures?

A fixture is the primary entity on the stage. It wraps physical hardware and adds stage-level attributes:

LED fixtures — linked to a performer child, with LED strings

DMX fixtures — linked to a DMX universe/address, with a profile and aim point

### Adding LED Fixtures

Go to Setup tab, click Discover to find performers

Click Add Fixture → select "LED" type

Link to a performer and configure strings (LED count, length, direction)

### Adding DMX Fixtures (Wizard)

Click + DMX Fixture on the Setup tab to launch the 3-step wizard:

Choose Fixture: Search the Open Fixture Library (700+ fixtures) or create a custom fixture

Set Address: Universe, start address, and name — with real-time conflict detection

Confirm: Review all settings, click "Create Fixture"

### DMX Monitor

Settings → DMX → DMX Monitor opens a real-time 512-channel grid per universe. Click any cell to set a value. Color-coded by intensity.

### Fixture Group Control

Settings → DMX → Group Control opens a control panel for fixture groups. Master dimmer slider, R/G/B sliders, and quick color preset buttons (Warm, Cool, Red, Off).

### Testing DMX Channels

On the Setup tab, click Details on any DMX fixture to open the channel test panel:

Sliders for every channel with live DMX output

Quick buttons: All On, Blackout, White, Red, Green, Blue

Capability labels show what each value range does (e.g., "Strobe slow→fast")

Changes take effect immediately on the physical fixture via Art-Net/sACN

### Fixture Types

| Type | Description |
| --- | --- |
| Linear | LED strip. Pixels along a path. |
| Point | DMX light source with beam cone. |
| Group | Collection of fixtures targeted as one. |

────────────────────────────────────────

## 5. Stage Layout

### 2D Canvas

The Layout tab shows a 2D front view of the stage. Stage dimensions (width × height) are set in Settings.

The layout toolbar provides: Save, 2D/3D mode toggle (shows current mode as text), Recenter, Top view, Front view, Auto-arrange DMX, Show/hide LED strings. Active toggles highlight in green.

Use the ?tab= URL parameter for deep-linking directly to any tab (e.g., ?tab=layout).

| Action | Desktop | Android |
| --- | --- | --- |
| Place fixture | Drag from sidebar | Tap fixture then tap canvas |
| Move fixture | Drag on canvas | Drag on canvas |
| Remove fixture | Double-click → Remove | Tap → Edit → Remove |
| Zoom | Scroll wheel | Pinch gesture |
| Pan | — | Two-finger drag |
| Edit coordinates | Double-click | Tap placed fixture |
| Edit object | Double-click | Tap object in list |

What's rendered:

Grid lines at 1-meter spacing

Stage dimension labels

LED fixtures: Green nodes with colored string lines (direction arrows)

DMX fixtures: Purple nodes with beam cone triangles toward aim point

Objects: Semi-transparent rectangles with name labels (clipped to stage)

Aim dots: Red circles at DMX aim points

### 3D Viewport (Desktop Only)

Toggle to 3D mode for an interactive Three.js scene:

Orbit camera with mouse drag

Beam cones as 3D geometry

Draggable aim spheres for DMX fixtures

Object planes/boxes with transparency

### Move / Rotate Mode

The layout canvas has two interaction modes, toggled with keyboard shortcuts or the toolbar button (which displays M or R to show the active mode):

| Key | Mode | Description |
| --- | --- | --- |
| M | Move | Drag any placed fixture to reposition it (default mode) |
| R | Rotate | Click a DMX fixture or camera fixture to show a compass ring; drag around the ring to aim the fixture |

Rotate mode details:

A compass ring appears around the selected fixture when you enter Rotate mode

Drag clockwise or counter-clockwise to set the horizontal aim direction

The beam cone updates in real-time as you drag

In 3D viewport, Rotate mode activates Three.js TransformControls in rotate mode — drag the colored arcs to rotate on any axis

Press Ctrl+Z to undo the last move or rotation

Typical workflow: Place all fixtures in Move mode, then switch to Rotate mode to aim DMX moving heads and cameras toward their intended focus areas before running calibration.

### Coordinate System

aimPoint[0] = X (horizontal position, mm)

aimPoint[1] = Y (height from floor, mm) — used for 2D canvas vertical axis

aimPoint[2] = Z (depth, mm) — used in 3D viewport only

canvasW = stage width × 1000 (mm)

canvasH = stage height × 1000 (mm)

────────────────────────────────────────

## 6. Stage Objects

Objects represent physical elements on the stage — walls, floors, trusses, screens, and props or performers — plus the abstract "targets" that Track actions chase. Anything a moving head needs to aim at lives in the Objects tab.

The tab was previously called "Surfaces"; the rename to Objects
shipped with v1.7.30. Old project files import cleanly.

### Object Types

| Type | Default mobility | Description |
| --- | --- | --- |
| Wall | Static | Back wall, stage-locked to stage width × height |
| Floor | Static | Stage floor, stage-locked to stage width × (depth + 1 m) |
| Truss | Static | Lighting truss bar |
| Screen | Static | Projection surface |
| Prop | Moving | Performer, set piece, or mobile element |
| Custom | Moving | User-defined object |
| Ribbon target | Moving | Travelling stage-coord anchor used by the Aurora Curtain template (#839) — a coordinated rig moves through the same point along a chosen axis |

### Stage-Locked Objects

Wall and floor objects can be locked to stage dimensions. When you change the stage size in Settings → Stage, locked objects resize automatically.

### Mobility

Static — fixed position. Cannot be tracked by moving heads.

Moving — position can change at runtime. Trackable by DMX moving

heads via the Track action (chapter 8).

### Patrol Motion

Moving objects can patrol during playback. Each patrol carries a pattern, an axis or shape, a cycle time, and an optional easing curve. The patrol evaluator runs at 40 Hz inside the DMX playback loop, immediately before Track actions read object positions, so a patrolling target's pose is always one frame fresh by the time a mover reads it.

#### Patterns

| Pattern | Geometry | Use it for |
| --- | --- | --- |
| Ping-pong | Travels from one bounding-box corner to the opposite, then reverses | A side-stage performer's predicted path |
| Circle | Constant-radius loop around the bounding-box centre | A turntable performer or a rotating stage gag |
| Figure-8 | Lemniscate (two-lobe) inside the bounding box | A complex path that visits two foci — useful for cross-stage chases |
| Square | Four straight-line legs around the bounding-box rim | A "patrol perimeter" feel used in security / industrial themes |
| Ribbon (new in v1.7.83) | A single travelling anchor on a chosen axis (left-right, front-back, up-down, cross, figure-8); multiple movers ride the ribbon at phase offsets so a sweep visibly travels the rig instead of every head moving in unison | The Aurora Curtain template's coordinated curtain effect |

#### Speed

Slow — 20 s cycle.

Medium — 10 s cycle.

Fast — 5 s cycle.

Custom — set cycleS directly. The default speedPreset is

medium; ribbon patrol objects ship with speedPreset: "custom" so the template's cycleS: 12 wins instead of the medium default.

#### Range

Bounding-box percentage (default 10 %–90 %). The patrol stays inside the box; targets near a wall don't lead a mover into IK gimbal lock.

#### Easing

Sine — smooth acceleration / deceleration (default).

Linear — constant velocity.

#### patrolMode

Patrol objects can be set to one of:

always (default) — patrol motion runs whenever the orchestrator

is up. Useful for "the prop is alive even before the show starts".

on-demand — patrol motion is suspended until an active timeline

references this object via a Track action. The instant a referencing clip starts playing, the patrol resumes from where it would have been if it had been running all along (so a Track action that expects a moving target sees one immediately, not a stationary prop). Operators are sometimes surprised to find an on-demand patrol prop sitting still on the stage view — that's the intended behaviour.

### Temporal Objects

External systems can create short-lived objects via POST /api/objects/temporal:

Always in-memory; never saved to disk.

Require ttl > 0 (time-to-live in seconds).

Auto-expire when TTL elapses.

Position updates refresh the TTL.

Shown in the runtime viewer with a dashed outline and a countdown

badge.

The camera tracker pushes detected people in via this route — each

detection becomes a temporal object that the Spotlight Follow Person preset's Track action chases.

The temporal-object scale uses the renderer's [width, height (Z), depth (Y)] ordering — call sites that want a "person-shaped" detection should ship [0.6, 1.8, 0.6] (60 cm wide, 1.8 m tall, 60 cm deep), not [0.6, 0.6, 1.8].

────────────────────────────────────────

## 7. Creating Spatial Effects

### Spatial Effects vs Classic Actions

Classic Actions (Solid, Chase, Rainbow, etc.): Run locally on each performer. Pattern based on pixel index. When assigned to DMX fixtures, classic actions are automatically converted to DMX Scene segments with appropriate dimmer, pan/tilt defaults.

DMX Actions: Control DMX-specific features directly:

DMX Scene — Set exact values for dimmer, pan, tilt, strobe, gobo, color wheel, prism

Pan/Tilt Move — Animate pan/tilt from start to end position over time

Gobo Select — Select a gobo wheel position

Color Wheel — Select a color wheel position

Track (Type 18) — Make moving heads follow moving objects in real-time (see Track Action)

Spatial Effects: Operate in 3D space. A sphere of light sweeping across the stage illuminates different fixtures at different times.

SlyLED supports 19 action types in total: 14 classic LED actions plus 5 DMX/spatial actions (DMX Scene, Pan/Tilt Move, Gobo Select, Color Wheel, Track).

### Creating a Spatial Effect

Navigate to Actions tab → + New Spatial Effect.

| Field | Description |
| --- | --- |
| Shape | Sphere, Plane, or Box |
| Color | RGB color applied to pixels inside the field |
| Size | Radius (sphere), thickness (plane), or dimensions (box) |
| Motion Start/End | 3D positions in millimeters |
| Duration | Travel time from start to end |
| Easing | Linear, ease-in, ease-out, ease-in-out |
| Blend | Replace, Add, Multiply, Screen |

────────────────────────────────────────

## 8. Track Action

The Track action (type 18) is the bridge between the Objects tab (chapter 6) and the moving-head rig: while a clip with a Track action is playing, every assigned moving head computes its pan / tilt aim from the live position of one or more target objects, every frame (40 Hz). When the clip ends — or the action's last referencing clip finishes — the assigned heads park at their Home pose (#807).

### How a Track action runs

Place the moving objects you want chased on the Layout / Objects

tab (chapter 6) — props, custom moving objects, ribbon targets, or camera-detected people coming in as temporal objects.

Create a Track action on the Actions tab. Set its name, scope,

and target list.

Drop a clip on a track in the Timeline that references the

Track action.

During playback, the 40 Hz DMX loop reads each target's current

position, picks (or assigns) one or more heads to chase it, runs the canonical aim-vector IK (#806 / #809), and writes pan / tilt to the universe buffer. The 3D viz's beam cone is driven from the same aim vector, so the visualisation always matches what the physical head is doing.

### Assignment algorithm

The assignment is recomputed every frame from trackFixtureIds (the heads in scope) and the resolved target list:

| Situation | Behaviour |
| --- | --- |
| Equal number of heads and objects | 1 : 1 mapping by index |
| More heads than objects, trackAutoSpread = false | Each object is chased by exactly one head; extra heads sit idle (still on, just not assigned). |
| More heads than objects, trackAutoSpread = true | Spread the heads across each object's width — useful for washing a single target with multiple movers. |
| More objects than heads, default | Cycle through objects with one head landing on a different target every trackCycleMs (default 2000 ms). |
| More objects than heads, trackFixedAssignment = true | Each head locks to one target by index; extras are ignored until a head frees up. |

Per-fixture offsets (trackFixtureOffsets) allow asymmetric rigs to aim each head at a slightly different point on the same target — for example, a stage-left mover aims at the performer's head while a stage-right mover aims at the feet.

### Action editor — Advanced expander (#811)

The Action editor's DMX Scene-family panes group the simple fields (name, scope, RGB, dimmer) at the top, with an Advanced expander underneath holding the per-action fine controls. For Track actions the Advanced section exposes:

| Field | What it controls | Range |
| --- | --- | --- |
| Cycle Time (ms) | trackCycleMs — only used when there are more objects than heads | 100 – 10 000 ms (clamped) |
| Offset X / Y / Z (mm) | Global aim offset added to every target. Useful for "aim 30 cm above the floor where the marker actually is" or "aim at head height instead of centre of mass". | ± 10 000 mm |
| Auto-spread | Toggles trackAutoSpread for the more-heads-than-objects case | — |
| Fixed assignment | Toggles trackFixedAssignment — disables cycling, locks each head to one target | — |
| Track dimmer | trackDimmer — overrides the dimmer the heads run at while the Track action is active. Defaults to 255 (full); operators sometimes lower this so the followed beam doesn't overpower the rest of the rig. | 0 – 255 |

### Target object selection

| Field | What it does |
| --- | --- |
| `trackObjectIds` | An explicit list of object IDs to track. Wins over trackObjectType when set. Searches the whole moving-object set (including patrol props), so an on-demand patrol prop's id resolves cleanly here. |
| `trackObjectType` | Filter moving objects by type — e.g. "prop", "ribbon-target", "custom". Useful when the target set is "every ribbon target" without listing IDs. |
| `trackMode` | Only consulted when trackObjectIds and trackObjectType are both empty. "camera-moving" (default) — track only camera-detected people (temporal objects). "all-moving" — track patrol props and camera detections together. |

If a Track action has explicit trackObjectIds but every listed object has been deleted, the action skips the frame entirely rather than blacking out the assigned heads — pre-fix (v1.7.78 and earlier) the missing-target case would zero the dimmer of every head in scope.

### Cooperation with shows and remotes (#763 / #835)

Track actions are full citizens of the show timeline:

An orphan Track action (one that lives in _actions but is not

referenced by any clip in the running timeline) does not evaluate (#835). This stops a leftover preset action from blacking out movers in unrelated timelines.

When a remote (Android phone or gyro controller) claims a head via the

mover-control claim arbiter (chapter on Remote control), the claimed head is muted from the Track action for the duration of the claim — operator gestures take priority over the show. Releasing the claim returns the head to the show without a re-bake.

Track actions write the canonical aim-vector through the same

_set_canonical_aim_stage path the rest of the orchestrator uses, so calibrate-end during a Track-driven show observes the head's true direction without an inverse-IK round-trip.

────────────────────────────────────────

## 9. Building a Timeline

Go to Runtime tab → + New Timeline

Set name and duration

+ Add Track for each fixture (or "All Performers")

+ Add Clip to assign effects with start time and duration

Clips can overlap — they blend according to their effect's blend mode

────────────────────────────────────────

## 10. Baking & Playback

### Bake

Compiles a timeline into minimal action instructions per performer:

Click Bake → progress shows frame count and segments

Click Sync to push instructions to performers via UDP

Click Start for synchronized NTP-timed playback

### Output

Action segments: Sequences of the 19 action types (14 classic + 5 DMX/spatial)

LSQ files: Raw per-pixel RGB data at 40Hz (downloadable as ZIP)

Preview data: 1 color per string per second for emulator

────────────────────────────────────────

## 11. Show Preview Emulator

Both desktop and Android include a real-time show preview:

### Dashboard Preview

When a show is running, the Dashboard tab shows a live stage preview canvas alongside the performer status table and playback progress bar.

### Desktop SPA

The emulator canvas appears on the Runtime tab below the timeline. Shows:

LED fixtures: Colored dots along string paths with glow effects

DMX fixtures: Beam cone triangles with preview-driven colors

Aim dots: Red circles at aim points

Fixture labels: Names below each node

Time counter: MM:SS elapsed / total

### Android App

The ShowEmulatorCanvas card shows:

Same LED string dots and DMX beam cones as desktop

Objects rendered as background rectangles

Preview colors update every second during playback

### Spatial Field Visualization

During show playback, the runtime emulator renders the active spatial effects moving across the stage:

Sphere: translucent colored circle moving along the motion path

Plane: translucent horizontal or vertical band sweeping across the stage

Box: translucent rectangle at the effect's current position

Effect names shown as labels at their current position

Updates every frame, synced to playback elapsed time

### DMX-Only Rigs

The emulator correctly renders DMX-only setups (no LED performers). Static purple beam cones always visible, with live colors when a show is running.

────────────────────────────────────────

## 12. DMX Fixture Profiles

### Built-in Profiles

| Profile | Channels | Features |
| --- | --- | --- |
| Generic RGB | 3 | Red, Green, Blue |
| Generic RGBW | 5 | Red, Green, Blue, White, Dimmer |
| Generic Dimmer | 1 | Intensity only |
| Moving Head 16-bit | 16 | Pan, Tilt, Dimmer, Color, Gobo, Prism |

### Profile Editor — Step-by-Step (#527)

The fixture profile editor maps a DMX channel to what it does — this red channel here, that pan channel there — and records any behaviour the fixture firmware expects on each channel (gobo slot ranges, strobe rate curves, colour-wheel slots). Once a profile exists the whole orchestrator can drive that fixture with semantic calls like "set colour to red" or "aim at stage (1150, 2100)" instead of raw DMX.

#### 1. Where to find it

Settings tab → Profiles sub-section. The list view shows every built-in and custom profile, filterable by category (par / wash / spot / moving-head / laser / effect). Each row has:

Edit — opens the editor on the selected profile (disabled for

built-in profiles; clone first if you want to diverge from one).

Clone — copy a built-in or community profile into your local

library under a new id; the copy is editable.

Share — upload a custom profile to the community server

(requires an internet connection, rate-limited per IP).

Delete — remove a custom profile (built-in profiles cannot be

deleted).

Click New Profile to start a fresh editor on a blank profile. You can also reach the editor from any DMX fixture card by clicking the profile name under the fixture's Edit Profile button.

#### 2. Top-level fields

Name — operator-visible label shown in fixture cards + the

profile picker.

Manufacturer — free text; used for grouping in the Community

browser and for dedup matching.

Category — par, wash, spot, moving-head, laser,

effect, other. Drives the preset-show generator.

Channel count — total DMX slots the fixture uses. Auto-updated

as you add channels; can also be set explicitly.

Colour mode — rgb, cmy, rgbw, rgba, single (monochrome

dimmer), color-wheel-only, or hybrid RGB + colour wheel. Drives how the show engine resolves a requested colour:

RGB family — the orchestrator writes the requested R / G / B

triple to the matching channels.

Colour-wheel-only — the orchestrator picks the closest wheel

slot for the requested RGB via the Euclidean-distance resolver described under Capabilities. Wheel slots without a hex annotation are skipped, so a profile that should match (say) red but doesn't has a missing color hex on its red WheelSlot capability.

Hybrid RGB + wheel — the profile carries BOTH a red/green/blue

triple AND a color-wheel channel. Since v1.7.83 (#842) the orchestrator drives RGB directly and parks the wheel at slot 0 (open / white) automatically, so the wheel's default coloured slot doesn't filter the beam. Pre-v1.7.83 hybrid fixtures often looked "wrong" because the wheel's home slot was tinting an RGB-driven colour; that's now handled at the lowest level (set_fixture_rgb) and every render path benefits.

Pan range / Tilt range — maximum mechanical sweep in

degrees. Used by mover calibration to normalise DMX→angle.

Beam width — degrees of the beam cone. Used for 3D beam-cone

rendering and for marker-coverage prediction.

#### 3. Channels

Every channel has:

Offset — 0-based channel number within the fixture's address

range (not the universe). A 16-channel fixture has offsets 0..15.

Name — operator-facing label. Matches the fixture's

documentation.

Type — the semantic role. Common types:

pan, pan-fine, tilt, tilt-fine, dimmer, red, green, blue, white, amber, uv, color-wheel, gobo, prism, focus, zoom, frost, strobe, macro, reset. The type is what downstream code reads when it wants to control "the dimmer" — you can rename the channel but the type is the contract.

Bits — 8 (one DMX slot) or 16 (two slots, coarse at this

offset + fine at offset+1). Use 16-bit for pan and tilt if the fixture supports it; everything else is typically 8-bit.

Default — the value the engine writes when no effect is

overriding the channel. Leave blank for "set to 0 at idle." Use a non-zero default for channels the fixture needs lit to function (e.g. a lamp-on macro, shutter-open slot).

#### 4. Capabilities

Each channel can carry a list of capabilities that describe what DMX value ranges mean to the fixture:

WheelSlot — colour or gobo wheel position. Range [min, max],

label ("Red", "Open", "Pattern 3"), and — for colour wheels — an optional `color` hex like #FF0000. The orchestrator's RGB→slot resolver (used by show bake and mover calibration) picks the closest-matching slot by Euclidean distance in RGB space, so every colour-labelled slot needs the hex filled in. Without the hex the RGB pipeline silently falls through to slot 0 (white/open), which is the #624 footgun.

WheelRotation — rotating-wheel range for cycle effects

("CW cycle fast-slow", "CCW cycle slow-fast").

WheelShake — jitter ranges on gobo wheels.

ShutterStrobe — a range with a shutterEffect of "Open",

"Closed", or "Strobe". The orchestrator's "open the shutter during calibration" helper walks these caps to find the right DMX value.

Prism, PrismRotation, Effect, NoFunction — same

pattern: range, label, optional type-specific fields.

Each capability row lets you pick the type from a dropdown, set min/max, add a label, and (for WheelSlot on colour wheels) a colour hex swatch.

#### 5. Saving + sharing

Save persists the profile to desktop/shared/data/dmx_profiles/

(gitignored per-install) and updates the SPA list.

Share to Community uploads the profile JSON to the electricrv.ca

server. The server dedups by channel-hash, so submitting a profile someone else already uploaded produces a "this fixture is already covered" response with a link to the existing entry.

Export downloads every custom profile as a single JSON bundle.

Use this to transfer a profile library between installs without going through the community server.

#### 6. When to create your own vs import from OFL

Import from OFL first — 700+ fixtures are already there, and

importing is one click. The Open Fixture Library volunteers have spent years curating the capability lists.

Clone and edit if the fixture is close to an OFL profile but a

channel or two differs (firmware update, mode variant).

Create from scratch only when the fixture is genuinely not in

OFL and not in the community. When you're done, share it so nobody else has to.

### Global brightness scaling (#843)

Master brightness reaches DMX fixtures through a per-frame scaling pass at render time, not through the bake. That keeps bakes valid across brightness changes — operators can drag the slider during a show without re-baking — and lets the Android Auto Brightness fast-path drive the master at ~20 Hz with the live envelope of stage audio.

Two cases:

Master-dimmer profiles — when the profile maps a dimmer

channel, the dimmer byte is scaled. The scaling is gamma-corrected (γ = 2.2 LUT) so a master setting of 50 % reads on a DMX par the same way it reads on an LED strip (which FastLED gamma-corrects internally).

RGB-only profiles — when there is no dimmer channel, the RGB

triple is gamma-scaled instead. Wheel-only fixtures fall back to scaling the dimmer (slot indices are categorical, not intensity).

The orchestrator broadcasts CMD_SET_BRIGHTNESS to every LED child on every change, and re-sends the current value to a child the first time it appears in PONG after a power cycle, so a freshly booted child doesn't display its first show frame at full intensity.

### Aim cone clamping (#803)

Aim requests on a moving head are clamped to the fixture's reachable cone, not rejected. If a Track action or /api/mover/<fid>/aim target lands outside the head's pan / tilt sweep, the orchestrator slides the aim along the cone rim toward the target instead of returning a 400 error. Operators see the head reach as far as it mechanically can; the 3D viz cone reflects the clamped aim, not the rejected one. Pre-v1.7.30 the operator-direct route would refuse the write, leaving the rig in a half-aimed state.

### Legacy quick-reference

Settings tab → Profiles → New Profile or Edit:

Define channels with name, type (red/green/blue/dimmer/pan/tilt/etc.), default value

Set beam width, pan/tilt range for moving heads

Import from Open Fixture Library (OFL) JSON format

### Browsing the Open Fixture Library

Click Search OFL in Settings → Profiles to access 700+ fixtures from the Open Fixture Library:

Search: Type a fixture name, manufacturer, or keyword → results show with Import buttons.

Browse by Manufacturer: Click Manufacturers to see all brands with fixture counts. Click a manufacturer to see all their fixtures. Click Import All to import every fixture from that manufacturer at once.

Bulk Import: From search results, click Import All to import all matching fixtures. From a manufacturer page, click Import All for the entire brand catalog.

Multi-mode fixtures create one SlyLED profile per mode automatically.

### Community Fixture Library

Share and discover profiles with other SlyLED users:

Browse: Click Community in Settings > Profiles to search, view recent, or popular

Download: Click Download — imported to your local library immediately

Share: Click Share on any custom profile to upload to the community

Dedup: Server detects duplicates by channel fingerprint (same channels = same fixture)

Unified search: When adding a DMX fixture, search queries Local + Community + OFL at once

Community server: https://electricrv.ca/api/profiles/

### Import/Export

Community: Share/download profiles with other users

Search OFL: Browse, search, and bulk import from the Open Fixture Library

Paste OFL: Paste raw OFL JSON for offline fixtures

Import Bundle: Load previously exported profile pack

Export: Download all custom profiles as JSON

Built-in profiles cannot be edited or deleted

────────────────────────────────────────

## 13. Preset Shows

Preset shows are a one-click way to put any rig into a polished look — useful for soundcheck, pre-show ambience, or a quick reset between cues. Open them from Runtime → Load Show → Presets.

The show generator inspects the rig at install time (LED strips, moving heads, camera nodes) and adapts each theme to what's actually on the stage. Themes that include moving-head choreography auto-classify any DMX fixture with panRange > 0 and tiltRange > 0 as a candidate, then emit either coordinated sweeps (stage-coordinate motion via ptStartPos / ptEndPos) or live target-tracking via a Track action.

### Available themes

| Preset | Description | Movers do |
| --- | --- | --- |
| Rainbow Up | Rainbow plane rising floor to ceiling | Stage-coord sweep up the Z axis (#837) |
| Rainbow Across | Rainbow sphere sweeping stage-left to stage-right | Stage-coord sweep along X |
| Slow Fire | Warm fire effect on every fixture | Static beam, dimmer flicker |
| Disco | Pastel twinkle sparkles | Mid-stage stage-coord chase |
| Ocean Wave | Blue wave sweep with teal wash | Slow front-to-back sweep |
| Sunset Glow | Warm breathe with golden sweep | Static aim, breathing dimmer |
| Police Lights | Red strobe with blue flash sweep | Strobe + slow side-to-side |
| Starfield | White sparkles on dark background | Static beam, dimmer twinkle |
| Aurora Borealis | Green curtain with purple shimmer | Slow front-to-back sweep |
| Aurora Curtain (new in v1.7.83) | Coordinated travelling curtain — every mover rides the same ribbon target at a phase offset, so the curtain visibly travels the rig instead of every head moving in unison. Includes a sparkle layer and fade-in / fade-out brackets so the look opens and closes cleanly. | Track a ribbon target, ping-pong on the chosen axis |
| Spotlight Follow Person | Warm orb that camera-tracked people inherit | Track action — heads chase detected person |
| Concert Wash | Magenta flood + amber sweep | Stage-coord sweep, no tracking |
| Figure Eight | Crossing orbs — heads trace X paths | Track action — heads chase crossing patrol props |
| Thunderstorm | Lightning strikes from the rig top-down | Strobe-style stage-coord bolts (Z-down, #837) |
| Dance Floor | Fast orbiting spots | Stage-coord chase, no tracking |

### Track-action vs sweep

The description column tells you whether the heads chase a target ("Track action") or follow a pre-baked sweep path ("stage-coord sweep"). Pre-v1.7.83, several themes promised tracking in their description but emitted only a sweep — fixed in #837 so the table is honest now: only Spotlight Follow Person, Figure Eight, and Aurora Curtain create a Track action when installed.

### Customising an installed preset

Installing a preset writes it into the regular Actions / Timeline / Objects state, so every part of it is editable in place:

Recolour a sweep — open the action under Actions, change RGB.

The bake re-renders next time the timeline plays; no need to re-install the preset.

Scope a Track action to specific fixtures — open the Track

action's Advanced expander (chapter 8), set trackFixtureIds to the heads you want, leave it empty to scope to every mover.

Adjust the tracker cycle — Cycle Time (ms) in the Advanced

expander controls how often a head jumps to a new target when there are more targets than heads.

Replace the patrol target — for Aurora Curtain or Figure Eight,

the underlying patrol object is on the Layout / Objects tab. Change its axis, speedPreset, or pattern; the next playback frame picks up the new motion.

### Behaviour outside playback

Preset shows respect the same idle-behaviour contract as any other timeline (chapter 17 → "Idle behaviour"): the rig parks at home and the lamp closes when the show ends, claim is released, or the operator stops playback. The Aurora Curtain ribbon's patrol object auto-stops at timeline end since it's tagged patrolMode: on-demand when the preset installs it (chapter 6).

────────────────────────────────────────

## 14. Camera Nodes

Camera nodes are Orange Pi or Raspberry Pi single-board computers with USB cameras. They provide live snapshots and AI-powered object detection for stage setup, mover calibration, and live person tracking via the Spotlight Follow Person preset.

The current production camera firmware is v1.6.3 (paired with orchestrator v1.7.83). Older deployed nodes can be updated in place from the Firmware tab — the SSH-deploy path picks up the new firmware bundle from dist/camera-firmware-v1.6.3.zip and replaces the camera service without rebooting the SBC.

Note: Only USB cameras are supported. Pi CSI ribbon cameras
(e.g. Pi Camera Module, Freenove FNK0056) are not supported. Use
USB webcams instead. Compatibility matrix for SBCs and USB sensors
lives in docs/SUPPORTED_HARDWARE.md — Orange Pi 4A is the primary
target; RPi 3B+ / 4 / 5 and Orange Pi Zero 3/5 are confirmed
working.

### Adding a Camera Node

Flash an Orange Pi with the supported OS image

Connect it to the same WiFi network as the orchestrator

In the Firmware tab, configure SSH credentials (default: root / orangepi)

Click Scan for Boards to find the device on the network

Click Install to deploy the camera firmware via SSH+SCP

### Camera Config Page

Each camera node serves a local web interface at http://<camera-ip>:5000/config:

Dashboard — board info, per-camera cards with live capture and detection

Settings — device name, reboot, factory reset

### Snapshots

Click Capture Frame on any camera card to take a JPEG snapshot. Uses OpenCV for fast capture with fswebcam fallback.

### Object Detection

Click Detect Objects (purple button) to run YOLOv8n AI detection on the current camera frame:

Bounding boxes with labels and confidence percentages are drawn on a canvas overlay

Threshold slider (0.1–0.9) — filter by detection confidence

Resolution (320/640) — lower is faster, higher is more accurate

Auto checkbox — continuously detect every 3 seconds

Typical latency: ~500ms capture + ~500ms inference on Orange Pi 4A

Detection requires the YOLOv8n ONNX model (models/yolov8n.onnx, 12 MB), which is uploaded automatically during firmware deployment.

### Camera Deploy

The deploy process (from the Firmware tab) uploads all camera files via SCP:

camera_server.py, detector.py, requirements.txt, slyled-cam.service

models/yolov8n.onnx (detection model)

Installs system packages (python3-opencv, python3-numpy, v4l-utils)

Installs Python packages (flask, zeroconf, onnxruntime)

Sets up the slyled-cam systemd service for auto-start on boot

Shows version comparison and supports force-reinstall

### Multi-Camera Support

Each node can host multiple USB cameras. The firmware auto-detects connected cameras and filters out internal SoC video nodes. Each camera gets its own card in the config page with independent capture and detection controls.

### Environment Scanning

The Scan Environment button on the Layout toolbar captures a 3D point cloud of the physical space:

Each positioned camera captures a frame and runs depth estimation

Pixels are back-projected to 3D using camera FOV and depth

Point clouds from all cameras are merged into stage coordinates

Surface analysis identifies floor, walls, and obstacles (pillars, furniture)

Detected surfaces can be automatically created as named stage objects

The point cloud can be viewed as colored dots in the 3D viewport (toggle with the point cloud button). This gives a visual map of the physical environment that the lights will illuminate.

### Per-Camera Fixtures

Each USB camera sensor on a camera node registers as a separate fixture in the layout. A node with 2 cameras creates 2 fixtures, each with:

Its own position on the stage (independently placeable)

Its own FOV and resolution

Its own rest direction vector (cyan arrow)

### Tracking Configuration

Each camera fixture has per-camera tracking settings accessible from the Edit dialog on the Setup tab. These control what the camera detects and how it behaves during live tracking.

Camera edit with tracking config

Detect Classes — Multi-select the object types to track. The YOLOv8n model supports 80 COCO classes; 16 stage-relevant classes are available:

| Category | Classes |
| --- | --- |
| People | Person |
| Animals | Cat, Dog, Horse |
| Props | Chair, Backpack, Suitcase, Sports Ball, Bottle, Cup, Umbrella, Teddy Bear |
| Vehicles | Bicycle, Skateboard, Car, Truck |

By default only Person is selected. Adding more classes has zero performance impact — YOLO always evaluates all classes in one pass and filters afterward.

Parameters:

| Parameter | Default | Range | Description |
| --- | --- | --- | --- |
| FPS | 2 | 0.5–10 | Detection frames per second. Higher = more responsive but more CPU on the camera node. |
| Threshold | 0.4 | 0.1–0.95 | Minimum confidence to accept a detection. Lower = more sensitive but more false positives. |
| TTL (s) | 5 | 1–60 | Seconds before a lost track expires and its stage marker is removed. |
| Re-ID (mm) | 500 | 50–5000 | Maximum distance to match a new detection to an existing tracked object. |

Starting tracking: Click the Track button on the Setup tab (next to Snap) or in the fixture edit modal on the Layout tab. The camera node begins continuous detection using your configured classes and parameters. Detected objects appear as labeled markers in the 3D viewport.

### Mover Calibration

The mover calibration wizard builds an interpolation grid that maps every stage position to the exact pan/tilt angles required for a DMX moving head. A positioned camera node is required.

Prerequisites:

At least one camera node positioned on the Layout tab

Art-Net engine running (POST /api/dmx/start)

Moving head fixture placed on the layout with its profile configured

Starting calibration:

Go to the Layout tab and double-click a DMX moving head fixture

Click the Calibrate button in the fixture edit dialog

Choose a beam color — options are Green, Magenta, Red, Blue (pick one that contrasts with your stage)

Click Start Calibration — the wizard takes over automatically

What happens automatically:

Discover — the head sweeps through a coarse pan/tilt grid; the camera detects where the beam lands on the stage floor

Map visible region — the pan/tilt range that keeps the beam within the camera's field of view is identified

Build interpolation grid — the head systematically samples points across the visible region; at each point the camera records the exact stage coordinates

Progress: A real-time progress panel shows the current phase, percentage complete, and a live thumbnail from the camera.

Result: The interpolation grid is saved to the fixture and used automatically by the Track action and any Pan/Tilt Move actions to convert stage-space coordinates into hardware pan/tilt values.

Tip: Run calibration in dim ambient lighting so the beam is clearly visible to the camera. Use the Beam Color option that gives the highest contrast against your floor surface.

### Fixture Orientation Test

Before running full calibration, use the orientation test to confirm pan and tilt are wired in the expected directions. Incorrect orientation causes calibration to converge on wrong positions.

Running the test:

Double-click a DMX moving head on the Layout tab to open the fixture edit dialog

Click Orientation Test (below the channel map)

The fixture moves through four probe positions: pan left, pan right, tilt up, tilt down

Watch the physical beam and compare with the on-screen arrows showing expected direction

Interpreting results:

| Observation | Action |
| --- | --- |
| Beam matches arrows | Orientation is correct — proceed to calibration |
| Pan moves opposite direction | Enable Invert Pan in the fixture settings |
| Tilt moves opposite direction | Enable Invert Tilt in the fixture settings |
| Pan and tilt axes are swapped | Enable Swap Pan/Tilt in the fixture settings |

Saving: After adjusting orientation flags, click Save in the fixture edit dialog. The flags are stored with the fixture and applied automatically during all subsequent calibration and playback.

────────────────────────────────────────

## 15. Firmware & OTA Updates

The Firmware tab is the operator's single window onto every flashable device on the rig: LED performers, the DMX-bridge, the gyro controller, and the camera nodes. Every flashable device reports its current firmware version up to the orchestrator on each PING/PONG cycle, so a stale device shows as "outdated" within seconds of the orchestrator booting.

### Current production versions (orchestrator v1.7.83)

| Device | Track | Current | Channel |
| --- | --- | --- | --- |
| Orchestrator (Windows / macOS) | app | v1.7.83 | installer (SlyLED-Setup.exe) |
| Android operator app | app | matches orchestrator track | sideload APK from dist/slyled-android.apk |
| LED Performer (ESP32) | child-led-esp32 | v7.5.11 | OTA |
| LED Performer (D1 Mini) | child-led-d1mini | v7.5.10 | OTA |
| LED Performer (Giga child) | child-led-giga | v7.5.2 | OTA |
| DMX Bridge (ESP32) | dmx-bridge-esp32 | v7.5.20 | OTA |
| DMX Bridge (Giga R1) | dmx-bridge-giga | v7.5.20 | OTA |
| Parent firmware (Giga R1) | parent-giga | v7.5.24 (on hold — desktop orchestrator is the recommended runtime) | USB only |
| Gyro Controller (ESP32-S3) | gyro-esp32s3 | v1.2.8 | OTA |
| Camera node (Linux SBC) | camera-node | v1.6.3 | SSH deploy from Firmware tab |

The Firmware tab queries firmware/registry.json to know what the "current" version is, so this table is regenerated automatically every release; the operator never has to keep it in their head.

### USB Flash

Go to the Firmware tab.

Click the USB Flash card. The dropdown lists every binary the

registry knows about for boards that flash over USB (LED ESP32 / D1 Mini / Giga child / DMX bridge variants / Gyro Controller).

Plug the target board in and pick its COM port from the second

dropdown.

Click Flash — progress shows percentage and a final

"verification OK" before the board reboots into the new firmware.

The Gyro Controller (ESP32-S3) ships with USB-CDC serial in the firmware. If a wedged build leaves the gyro unable to enumerate over USB, hold the BOOT button while plugging in to enter the manual ROM bootloader; the Firmware tab then re-flashes through esptool's recovery path.

### OTA (Over-the-Air)

Set WiFi credentials on the Firmware tab — these get pushed to every

newly-flashed device.

Click Check for Updates. The tab shows a per-device comparison:

reported version → registry version, with an Update button on anything outdated.

Click Update on any outdated performer. Mid-flash status comes

back live; the device reboots automatically after verification.

New since v1.7.83: when a registry SHA-256 mismatches the on-disk

binary (a download mid-update or a hand-edited registry), the orchestrator falls back to the GitHub release for that board's releaseTag rather than refusing the flash.

The diagnostic / development gyro builds (esp32s3-gyro-test-firmware.bin) are deliberately hidden from the operator OTA UI — the Firmware tab only offers production builds. The diagnostic binary is still present in dist/ for engineers running paired-board debugging.

### Firmware Registry

firmware/registry.json is the single source of truth for what version the orchestrator believes ships with each release. Each entry carries:

id and name for the OTA UI label.

version (3-part semver) — what the operator's device should be

running.

releaseTag and releaseAsset — the GitHub release tag and the

asset filename inside it, used by the OTA fallback.

sha256 — verification hash that the orchestrator checks before and

after flashing.

Editing registry.json by hand is not recommended; build_release.ps1 keeps it in sync with the actual binary hashes on every release. The Firmware tab refreshes the registry from GitHub on demand from the Refresh button so a freshly-pulled installer immediately sees the versions corresponding to its release tag.

────────────────────────────────────────

## 16. System Limits

| Resource | Tested | Recommended Max |
| --- | --- | --- |
| DMX fixtures | 120 | 500+ |
| LED performers | 12 | 50 |
| Total fixtures | 132 | 500+ |
| Universes | 4 | 32,768 (Art-Net) |
| LEDs per string | 65535 | uint16 addressing |
| Strings per child | 8 | Protocol constant |
| Timeline clips | 50 | 200+ |
| Preset shows | 14 | Built-in (expandable) |
| API response (132 fixtures) | < 1ms | Sub-millisecond |
| Memory (132 fixtures) | 46 MB | Flat scaling |
| Network (132 fixtures) | 221 KB | Per test cycle |

See docs/STRESS_TEST.md for full benchmark data.

────────────────────────────────────────

## 17. Troubleshooting

The table below covers the symptoms operators most often raise. Every row points to the orchestrator version where the underlying behaviour was last touched, so an operator running an older build can decide whether to update or work around.

| Problem | What you'll see | Fix or version |
| --- | --- | --- |
| Runtime view empty | The 3D Runtime tab shows the stage but no fixtures. | Check that fixtures are positioned in the Layout tab. DMX-only rigs render correctly since v1.7.30. |
| Beam cone wrong direction | The 3D viz cone aims at the wrong wall. | Beam direction comes from the fixture's rotation = [rx, ry, rz] in stage space. Z is up; rx > 0 aims down. See chapter 4 for the full convention. |
| 3D viz cone disagrees with the physical head | Cone in the viz points stage-left but the moving head is aimed stage-right. | Fixed in v1.7.52 (#806/#809): the canonical aim vector is the source of truth and the physical IK derives from it. If you still see the disagreement on v1.7.52+, re-save the fixture's Home and Secondary in the Set Home wizard. |
| Calibrate-end pan jump | Pressing release on calibrate snaps the head to a different pose than the gyro was reporting. | Fixed in v1.7.52 (#805). Pre-fix the legacy IK fallback was capturing the wrong aim vector at release time. Operators on v1.7.52+ who still see a jump should report it with the gyro firmware version (must be ≥ v1.2.4). |
| Press Start blinks back to "start" on the gyro | Operator presses Start after a WiFi gap, the gyro UI flashes claim-acknowledge for a frame, then reverts to IDLE while the orchestrator holds an orphan claim. | Fixed in v1.7.83 (#812 / #813 / #825). Press-Start now uses a 16-bit nonce + CLAIMACK, with HBREP heartbeats to reconcile divergent state. If you see the symptom on v1.7.83+, check that the gyro firmware is ≥ v1.2.7 (registry will warn). |
| Auto Brightness has no effect on the lights | The Android Auto Brightness UI shows the master sliding with the music, but DMX heads and LED strips don't dim. | Fixed in v1.7.83 (#843). The fast-path POST now broadcasts CMD_SET_BRIGHTNESS to LED children and gamma-scales DMX dimmer / RGB at render time. Operators on older builds can fall back to the manual Settings → Global Brightness slider until they update. |
| Looping playlist blacks out between iterations | A single-item or multi-item playlist set to Loop All flashes everything to zero for one frame at every wrap. | Fixed in v1.7.83 (#840). Single-item loops route through the modulo-wrap playback path; multi-item loops pass is_final=False to suppress the natural-end blackout sweep until the playlist actually stops. |
| Track action blacks out movers in unrelated timelines | A timeline that doesn't reference a particular Track action still has its movers go dark whenever that action exists in the action library. | Fixed in v1.7.83 (#835). Track actions now only evaluate on timelines that reference them; orphan actions stay dormant. |
| Show preset says "moving heads track / follow / chase X" but they don't | A theme description promises tracking, but the rig just sweeps. | Fixed in v1.7.83 (#837). Theme descriptions now match the actual implementation: only Figure Eight and Spotlight Follow Person emit a Track action; the others sweep. |
| Colour Wheel field appears in DMX Scene action editor | The action editor's DMX Scene / PT-Move / Gobo Select panes had a "Colour Wheel" input that didn't do what the operator expected on hybrid RGB+wheel fixtures. | Fixed in v1.7.83 (#841 / #842). The Colour Wheel slot is now type-17 only; the bake/render layer derives the slot from RGB via rgb_to_wheel_slot for every other action type. A one-shot migration strips stale colorWheel: 0 fields on first start of v1.7.83+. |
| 3D viewport not rendering | Black canvas where the stage should be. | Use Chrome / Firefox / Edge with WebGL support. Check chrome://gpu for hardware acceleration. |
| Performers not syncing | A child shows offline in Setup but is powered up. | Check that the orchestrator and the child are on the same WiFi subnet. The Setup tab's Refresh rescans via mDNS + UDP broadcast. |
| Canvas wrong size | The Layout canvas is much smaller or larger than the room. | Stage dimensions (Settings → Stage) drive canvas size: canvasW = stage.w × 1000. Adjust stage width/height in metres rather than canvas pixels. |
| OTA flash refused with SHA mismatch | Firmware tab refuses to update with sha256 mismatch. | Fixed in v1.7.61 (#814). The orchestrator now falls back to the GitHub release for the registered releaseTag when the on-disk binary disagrees with the registry. If you still see this, click Refresh on the Firmware tab to re-fetch registry.json from GitHub. |
| Gyro stale-reason latch never clears | "Connection lost" stays on a gyro status row even after the gyro resumes streaming. | Fixed in v1.7.62 (#821) and again in v1.7.63 (#823). Press-Start clears the remote stale_reason; cache self-destructs on a transient read failure. |

If you hit something not in this table, the orchestrator's log (Settings → Logging → enable file logging) captures every UDP send and DMX render decision tagged by issue number — open an issue on GitHub with the relevant section attached and a description of what the rig was doing at the moment.

────────────────────────────────────────

## 18. Examples

### Example A: Camera Tracking — Moving Heads Follow a Person (#376)

Make DMX moving heads automatically follow people detected by a camera.

Prerequisites:

At least one camera node online (Firmware tab → deploy + verify)

At least one DMX moving head fixture placed on the Layout tab

Moving head profile configured with pan/tilt range

Art-Net/sACN engine running (Settings → DMX → Start)

Mover calibration completed (see Example C) for accurate aiming

Steps:

Verify hardware — Open the Setup tab. Confirm your movers show green status and camera nodes show online. If cameras are offline, check WiFi and deploy firmware from the Firmware tab.

Start camera tracking — Click the Track button on the Setup tab (next to Snap), or go to the Layout tab, click a camera fixture, and click the Track button in the edit modal. The camera node begins running YOLO detection using the classes and parameters configured in the camera's tracking settings (see Tracking Configuration). Detected objects appear as labeled markers in the 3D viewport.

Create a Track action — Go to the Actions tab. Click + New Action.

Name: Person Follow

Type: Track (last option in the dropdown)

Colour: Pick the beam color (e.g. red for a spotlight)

Leave Target Objects empty — this means "follow ALL detected people"

Cycle Time: 2000 ms (how fast heads switch if cycling)

Check Fixed assignment if you want strict 1:1 (head 1 = person 1, extras ignored)

Click Save Action

Create a timeline — Go to the Shows tab. Click + New Timeline, name it "Person Tracking", set duration to 600s, enable Loop. The timeline can be empty — Track actions evaluate globally during any playback.

Start playback — Click Bake, then Start. The 40 Hz DMX playback loop begins. The Track action reads all moving temporal objects (detected people), computes pan/tilt for each head, sets dimmer and color, and sends Art-Net packets to the bridge.

Test — Walk in front of the camera. Within 2 seconds a pink person marker appears in the 3D viewport. The moving heads should light up in your chosen color and aim at you.

Assignment behavior:

| People in view | With 2 moving heads |
| --- | --- |
| 1 person | Both heads aim at the same person |
| 2 people | One head per person (1:1) |
| 3+ people (cycling) | Heads cycle through people every 2s |
| 3+ people (fixed) | First 2 tracked, 3rd ignored |

Troubleshooting:

| Problem | Solution |
| --- | --- |
| No person markers in 3D | Check camera node status — is tracking running? Try a manual Scan to verify detection works. |
| Person detected but heads don't move | Check Art-Net engine is running. Check mover calibration. Verify timeline playback is active. |
| Heads light up but aim at wrong position | Run mover calibration (Example C). Without calibration, the system uses geometric estimates which may be inaccurate. |
| Heads respond with delay | Normal — detection runs at 2 fps with ~1s capture latency. Temporal objects have 5s TTL. |

────────────────────────────────────────

### Example B: Mover Tracking with Spatial Effects (#379)

Make moving heads follow a virtual object sweeping across the stage — no camera required. This example walks through the complete workflow from stage setup to live 3D preview with animated beam cones.

Prerequisites:

SlyLED orchestrator running (Windows or Mac)

No physical hardware required — this example runs entirely in the emulator

Part 1 — Stage and Fixture Setup

Set stage dimensions — Open the Settings tab. Under Stage, enter the dimensions of your performance area:

Width: 6000 mm (6 m)

Height: 3000 mm (3 m)

Depth: 4000 mm (4 m)

Click Save. The 3D viewport will resize to match these dimensions.

Create a DMX profile — Go to Settings → Profiles → click New Profile. This defines the channel layout of your moving head:

Name: Narrow Spot

Beam Width: 8 (degrees — narrow beam for visible tracking)

Pan Range: 540, Tilt Range: 270

Channels: Add 6 channels in this order:

Ch 0: Pan (16-bit) — pan coarse

Ch 1: Pan Fine — pan fine (auto-linked)

Ch 2: Tilt (16-bit) — tilt coarse

Ch 3: Tilt Fine — tilt fine (auto-linked)

Ch 4: Dimmer

Ch 5: Red, Ch 6: Green, Ch 7: Blue

Click Save Profile

Profile editor with narrow spot configuration

Add two movers — Go to the Setup tab. Click + Add Fixture twice to create two DMX moving heads:

Fixture 1: Name: Mover SL (Stage Left), Universe: 1, Start Address: 1, Profile: Narrow Spot

Fixture 2: Name: Mover SR (Stage Right), Universe: 1, Start Address: 14, Profile: Narrow Spot

Both fixtures appear in the Setup table with purple "DMX" badges and the profile name.

Part 2 — 3D Layout and Spatial Effect

Position movers on the truss — Switch to the Layout tab. In the sidebar, you'll see both movers listed as "unplaced." Drag each one into the 3D viewport:

Mover SL: Position at X: 1500, Y: 0, Z: 2800 (stage left, on truss). Set rotation to tilt: -30, pan: -15.

Mover SR: Position at X: 4500, Y: 0, Z: 2800 (stage right, on truss). Set rotation to tilt: -30, pan: 15.

Switch to 3D view to confirm both movers are elevated on the truss and aimed downward toward the stage floor. The beam cones should be visible as translucent triangles.

Layout tab — 3D view with two movers positioned on truss

Create a spatial effect — Go to the Actions tab. Click + New Action:

Name: Sweep Green

Type: Spatial Effect

Shape: Sphere

Radius: 800 mm

Color: Green (0, 255, 0)

Motion Start: X: 1000, Y: 2000, Z: 0 (stage left, mid-depth, floor level)

Motion End: X: 5000, Y: 2000, Z: 0 (stage right, same depth and height)

Duration: 8 seconds

Easing: Linear

Click Save Action

This creates a green sphere of light that sweeps from stage left to stage right over 8 seconds. When applied to moving heads, they will track the sphere's center position.

Actions tab with Sweep Green spatial effect configured

Part 3 — Timeline, Bake, and Playback

Create a timeline — Go to the Shows tab. Click + New Timeline:

Name: Mover Tracking Demo

Duration: 20 seconds

Loop: Enabled

Add a track targeting All Performers

Add a clip referencing the Sweep Green effect, starting at 0s with 8s duration

Shows tab with timeline containing the spatial effect clip

Bake the timeline — Click the Bake button. The bake engine computes per-fixture pan/tilt angles for each time slice:

For each 25ms frame, it calculates the sphere's position along the motion path

For each mover, it computes the pan/tilt angles needed to aim at that position

Dimmer is set to 255 and color channels are set to green

Wait for "Bake complete" confirmation (typically < 1 second)

Start playback and verify — Switch to the Runtime tab. Click Start:

The 3D viewport shows both beam cones animated in real-time

At T=0s, both beams aim at the starting position (stage left)

As the effect sweeps, the beams track the green sphere across the stage

At T=8s, both beams have followed the sphere to stage right

The timeline loops, and the sweep restarts

Runtime — beam cones at start position (T=0s)

Runtime — beams tracking mid-sweep (T=5s)

Runtime — beams at end position (T=10s)

What to look for:

Both beam cones should be green (matching the effect color)

The cones should move smoothly from left to right

The beam intensity (opacity) should be > 0 during the sweep, indicating active output

If beam cones don't appear, ensure fixtures are positioned in the Layout tab and the timeline is baked

Variations:

Change the spatial effect shape to Plane for a wall of light sweeping across

Add a second effect on a separate track with different timing for crossing patterns

Try the Figure Eight preset show (Runtime → Load Show) for a ready-made crossing pattern

────────────────────────────────────────

### Example C: Manual Mover Calibration (#381)

Calibrate a moving head so the system knows exactly where its beam lands for any pan/tilt position. This two-part process first discovers the beam's visible range (pan/tilt grid) and then builds a light map that maps every pan/tilt position to real stage coordinates.

Prerequisites:

At least one camera node online and positioned on the Layout tab

Camera calibration complete — the camera must have a valid stage map (see Example D)

Moving head fixture added in Setup and positioned on the Layout tab

Art-Net/sACN engine running (Settings → DMX → Start)

Dim ambient lighting — the beam must be clearly visible to the camera against the floor

The beam should be aimed at the floor within the camera's field of view, not directly at the camera

Part 1 — Pan/Tilt Discovery and Grid Calibration

Open the calibration panel — Go to the Layout tab. Double-click the moving head fixture you want to calibrate. In the edit dialog, click the Calibrate button. The calibration wizard opens showing the fixture name, current calibration status (if any), and available calibration modes.

Calibration panel before starting — shows fixture name and calibration options

Choose beam color — Select a color that contrasts well with your floor surface:

Green works best on dark floors (wood, dark carpet)

Magenta works best on light floors (white, concrete)

Red or Blue are alternatives if the default choices blend with your environment

The color matters because the camera uses color filtering to isolate the beam from ambient light

Run discovery — Click Start Calibration. The system runs an automatic discovery sequence:

Phase 1 — Coarse grid scan: The fixture sweeps through ~40 pan/tilt positions (8 columns x 5 rows) across its full range. The camera watches for the beam appearing on the floor after each move.

Phase 2 — Fine refinement: Once the beam is found, the system spirals outward from that position to refine the exact center of the visible region.

Discovery typically completes in 30-60 seconds. The progress indicator shows "Discovering..." with the current scan position.

Discovery in progress — coarse grid scan with camera watching for beam

BFS mapping — After discovery, the system automatically maps the full visible region:

Starting from the discovered beam position, it steps in 4 directions (up/down/left/right in pan/tilt space)

At each position, the camera captures a frame and detects the beam centroid

The system records the beam's pixel position and converts it to stage millimeters using the camera's homography

Mapping stops at boundaries where the beam leaves the camera's field of view or falls off the stage

Collects up to 60 sample positions, typically completing in 2-3 minutes

The system uses adaptive settle times (0.8-2.5s) per move and double-capture verification to ensure the beam has stopped before recording

Grid build and review — The collected samples are compiled into a bilinear interpolation grid:

The calibration summary displays:

Sample count: Number of successfully detected positions (aim for 30+)

Pan range: Normalized range (e.g., 0.15-0.85 means the beam is visible across 70% of pan range)

Tilt range: Normalized range

Grid density: How finely the grid was sampled

The grid enables fast forward lookup: given a (pan, tilt) value, compute the stage (X, Y) where the beam lands

Grid calibration complete — summary showing sample count, pan/tilt range, and grid density

Part 2 — Light Map Calibration (stage coordinates to pan/tilt lookup)

Build the light map — Click Build Light Map. This extends the calibration by sweeping a systematic 20x15 grid across the discovered visible region:

For each grid position, the fixture moves to the pan/tilt value

The camera detects the beam and records the exact stage X/Y/Z where it lands

This builds a comprehensive (pan, tilt) → (stageX, stageY, stageZ) lookup table

Progress shows as "Building light map... N/300" with real-time updates

Typical completion time: 5-10 minutes for a full 20x15 grid

Light map build in progress — systematic sweep with stage coordinate mapping

Verify inverse lookup — After the light map is built, use the Aim button to test the inverse mapping:

Enter a target stage position (e.g., center stage: X=3000, Y=2000, Z=0)

Click Aim — the system uses inverse-distance weighted interpolation of the 4 nearest light map samples to compute the exact pan/tilt values

The fixture moves to the computed position

Verify visually that the beam lands on (or very near) the target point on stage

Try 3-4 different targets across the stage to confirm accuracy

Good calibration should place the beam within 100-200mm of the target at typical stage distances

Aim verification — beam aimed at target stage position using calibrated light map

Save calibration — Calibration data is automatically saved with the fixture. The light map and grid data persist across sessions and are included in project file exports (.slyshow).

Track actions use the light map to aim at detected people

Pan/Tilt Move actions use it for smooth interpolated sweeps

The 3D viewport uses it to render accurate beam cone directions

Manual calibration (alternative — no camera required):

If automated calibration isn't available (no camera, or camera can't see the beam), use the manual calibration wizard:

Layout tab → double-click mover → click Manual Calibrate

Define marker positions — Add 4-6 physical markers at known stage positions. Enter each marker's X, Y, Z coordinates (in mm). Spread markers across the stage: front-left, front-right, back-center at minimum.

Jog to each marker — For each marker, use the pan/tilt sliders to manually aim the beam until it lands exactly on the physical marker. Click Record to save the (pan, tilt) → (stageX, stageY, stageZ) sample.

Add at least 4 samples spread across the stage for a good affine fit. More samples (6+) improve accuracy, especially at stage edges.

Click Compute — the system fits a 3D affine transform from your samples:

pan = a1*stageX + b1*stageY + c1*stageZ + d1

tilt = a2*stageX + b2*stageY + c2*stageZ + d2

The affine transform extrapolates beyond calibrated points for full-stage coverage

When to re-calibrate:

Fixture physically moved to a new position or angle

Venue change (different stage dimensions or floor surface)

After firmware update that changes pan/tilt range or motor behavior

If aim accuracy degrades over time (motor drift)

After changing the fixture's mounting orientation (upright vs. inverted)

────────────────────────────────────────

### Example D: Camera Calibration with ArUco Markers (#380)

Calibrate a camera so pixel coordinates can be mapped to real stage positions. This is a prerequisite for beam detection, person tracking, and mover calibration — without it, the system cannot convert what the camera sees into real-world stage millimeters.

Prerequisites:

Camera node online and reachable on the network (deploy firmware from the Firmware tab if needed)

Camera fixture registered in the system (Setup tab → Discover, or Settings → Cameras → add manually)

Camera fixture placed on the Layout tab at its physical position

A printer to print the ArUco marker sheet (standard A4/Letter paper)

A tape measure to record marker positions on stage

The camera must have a clear view of the stage floor where markers will be placed

Part 1 — Prepare and Place ArUco Markers

Print ArUco markers — Go to Settings → Cameras. Click the Print ArUco Markers button. A modal opens with 6 printable ArUco 4x4 markers (IDs 0-5), each 150mm x 150mm:

Click Download or use the browser's print dialog to print the marker sheet

Print at 100% scale (no scaling/fit-to-page) — the physical size must match the expected 150mm for accurate calibration

Markers can be printed on regular white paper, but card stock is more durable

ArUco marker print dialog — 6 markers ready to print

Place markers on the stage floor — Position the printed markers at known locations on the stage:

Minimum: 3 markers (enough for a basic homography)

Recommended: 4-6 markers for better accuracy

Placement strategy:

Spread markers across the entire camera's field of view

Place at least one marker near each corner of the visible area

Place markers flat on the floor — tilted markers reduce accuracy

Measure each marker's position from the stage origin (back-right corner at floor level):

X = distance from stage right (mm)

Y = distance from back wall (mm)

Z = 0 (floor level)

Record the marker ID and its (X, Y) coordinates — you'll enter these in step 5

Part 2 — Register and Position the Camera

Register the camera — If the camera node is not already registered:

Go to the Setup tab and click Discover — camera nodes respond to UDP broadcast

Or go to Settings → Cameras → enter the camera's IP address manually

Each USB camera sensor on the node appears as a separate fixture

Verify the camera is online: its status should show "Online" with a green indicator

Camera configuration panel in Settings — camera list with IP, status, and calibration badges

Position the camera in 3D — Switch to the Layout tab:

Find the camera fixture in the sidebar (listed as "unplaced" if new)

Drag it into the 3D viewport at the camera's real physical position

Set the rotation to match the camera's actual aim direction:

A camera mounted on a wall at 2m height, aimed down at 30 degrees would have rotation Z=2000, tilt=-30

In 3D view, the camera appears as a frustum (pyramid) showing its field of view

Verify the frustum covers the area where you placed the ArUco markers

Part 3 — Run Calibration and Verify

Run ArUco calibration — In the Layout tab, click on the camera fixture to select it. Click the Calibrate button:

The wizard opens and fetches a live snapshot from the camera

The system automatically detects all visible ArUco markers and highlights them with green overlays

For each detected marker:

The marker ID is shown on the overlay

Enter the marker's real-world stage coordinates (X, Y in mm) that you measured in step 2

Click Record to save the pixel-to-stage mapping for this marker

After recording all markers, click Compute — the system builds a homography matrix that maps any pixel coordinate to stage floor coordinates

Camera snapshot with ArUco markers detected — green overlays showing marker IDs

Review calibration results — The calibration summary shows:

Reprojection error: How accurately the computed homography matches the recorded points. Lower is better:

< 10mm: Excellent — suitable for precision tracking

10-20mm: Good — adequate for most use cases

20-50mm: Fair — consider adding more markers or re-measuring positions

> 50mm: Poor — re-check marker measurements and try again

Reference points: Number of markers used (should match what you recorded)

Coverage area: The stage area covered by the calibration (larger is better)

Calibration complete — reprojection error, reference points, and coverage summary

Save and apply — Click Save to persist the calibration:

The camera fixture badge updates to show a green "Cal" checkmark

All features that depend on pixel-to-stage conversion now use this calibration:

Person tracking: Detected bounding boxes are converted to stage positions

Beam detection: Beam centroids become stage coordinates for mover calibration

Mover calibration: The entire mover calibration wizard (Example C) requires this

Calibration data is included in project file exports (.slyshow) for portability

Tips for accurate calibration:

Marker size matters: Use the 150mm markers at standard print size. Smaller markers are harder to detect at distance.

Flat placement is critical: Even a slight tilt (marker on a crumpled surface) can shift the detected center by 10-20mm.

Cover the edges: The homography is most accurate within the convex hull of your reference markers. Place markers at the extremes of the camera's view, not just the center.

Lighting conditions: ArUco detection works in most lighting, but avoid direct glare on the printed markers (glossy paper under bright lights).

Re-calibrate when:

The camera is physically moved (even slightly)

The camera lens is changed or zoom is adjusted

Stage dimensions change (markers would be at different positions)

Accuracy of tracking or beam detection degrades

────────────────────────────────────────

### Example E: Spotlight Follow Person — Live Tracking Preset (#382)

Use the built-in Spotlight: Follow Person preset to make moving heads automatically follow people detected by a camera in real-time.

Prerequisites:

At least one camera node online with person detection working (verify with a manual Scan first)

At least one DMX moving head fixture placed on the Layout tab

Camera calibration complete (see Example D) for accurate stage positioning

Mover calibration complete (see Example C) for accurate pan/tilt aiming

Art-Net/sACN engine running

Steps:

Load the preset — Go to the Runtime tab. Click Load Show (or the preset dropdown). Select Spotlight: Follow Person from the preset list.

If no camera node is registered, a warning appears: "No camera node registered — person detection will not work"

If no moving heads are configured, a warning appears about missing movers

The preset loads even with warnings — you can add the missing hardware later

What it creates — The preset automatically configures:

A Track action (type 18) on every available moving head, targeting objectType: "person"

A warm spotlight color (255, 240, 200) at full dimmer for the beam

A dim blue ambient wash (10, 5, 30) on all LED fixtures for atmospheric framing

A 10-minute looping timeline that keeps the DMX playback loop running

Start camera tracking — Click the Track button on the Setup tab or in the camera fixture edit modal. The camera node begins running detection using the configured tracking classes and parameters (see Tracking Configuration). Detected objects appear as labeled markers in the 3D viewport.

Start playback — Click Bake, then Start. The 40 Hz DMX playback loop begins. The Track action reads all temporal person objects and computes pan/tilt for each head in real-time.

Walk on stage — Within 2 seconds of entering the camera's view, a pink person marker appears. Moving heads light up with the warm spotlight color and aim at you. As you move, the beams follow.

Behavior with multiple people:

1 person, 2 heads: Both heads aim at the same person

2 people, 2 heads: One head per person (auto-spread)

3+ people, 2 heads: Heads cycle through people every 2 seconds

When no one is detected:

Heads dim to 0 (blackout) and hold their last position

As soon as a person is detected again, heads immediately re-aim and light up

Tips:

Use a narrow beam profile (8-15 degrees) for a dramatic spotlight effect

Ensure the room is dim enough for the camera to distinguish the beam from ambient light

If tracking seems jittery, increase the camera's capture FPS or reduce the confidence threshold

The Track action works alongside other timeline effects — you can add spatial color washes on lower-priority tracks

────────────────────────────────────────

## 19. API Quick Reference

### Stage & Layout

| Method | Endpoint | Description |
| --- | --- | --- |
| GET/POST | /api/layout | Layout with fixtures and positions |
| GET/POST | /api/stage | Stage dimensions (w, h, d meters) |
| GET/POST | /api/objects | Stage objects (walls, floors, trusses, props) |
| POST | /api/objects/temporal | Create temporal objects (TTL-based) |

### Fixtures

| Method | Endpoint | Description |
| --- | --- | --- |
| GET/POST | /api/fixtures | List / create |
| GET/PUT/DELETE | /api/fixtures/:id | CRUD |
| PUT | /api/fixtures/:id/aim | Set aim point |

### Shows & Timelines

| Method | Endpoint | Description |
| --- | --- | --- |
| GET/POST | /api/timelines | List / create |
| POST | /api/timelines/:id/bake | Start baking |
| POST | /api/timelines/:id/start | Start playback |
| GET | /api/show/presets | List preset shows |
| GET/POST | /api/show/export, /api/show/import | Save/load show file |

### DMX

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/dmx-profiles | List profiles |
| GET | /api/dmx/patch | Universe address map |
| POST | /api/dmx/start, /api/dmx/stop | Engine control |

### Cameras

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/cameras | List registered camera fixtures |
| POST | /api/cameras | Register a camera node as fixture |
| DELETE | /api/cameras/:id | Remove camera fixture |
| GET | /api/cameras/:id/snapshot | Proxy JPEG snapshot |
| GET | /api/cameras/:id/status | Live status from camera node |
| POST | /api/cameras/:id/scan | Object detection (proxy to node /scan) |
| GET | /api/cameras/discover | Find camera nodes on network |
| GET/POST | /api/cameras/ssh | SSH credentials for deployment |
| POST | /api/cameras/deploy | Deploy firmware to camera node via SSH+SCP |
| GET | /api/cameras/deploy/status | Poll deploy progress |

### Camera Node Local API (port 5000)

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /status | Node status, capabilities, camera list |
| GET | /config | HTML config page with detection UI |
| GET | /snapshot?cam=N | JPEG snapshot from camera N |
| POST | /scan | Object detection (JSON: cam, threshold, resolution, classes) |
| GET | /health | Health check |

────────────────────────────────────────

## 20. Glossary

SlyLED touches lighting, networking, computer vision, and embedded firmware — which means a lot of acronyms and jargon. This section expands every acronym used elsewhere in the manual and defines the domain terms that don't have a literal expansion ("baking", "universe", "blink-confirm", etc.).

Entries are alphabetised on the Term column. For acronyms that cluster around a common concept (e.g. RX / RY / RZ) the cluster appears under the first member.

| Term | Expansion | Plain-language definition | Where it shows up |
| --- | --- | --- | --- |
| API | Application Programming Interface | The set of HTTP endpoints a program exposes for other programs to call. | §19 API Quick Reference; /api/* routes throughout. |
| ARM | Advanced RISC Machine | CPU architecture used by the Giga R1, Raspberry Pi, and Orange Pi. "Slow on ARM" in the manual means these boards. | §14 Camera Nodes (depth-estimation runtimes). |
| Art-Net | — | DMX-over-Ethernet protocol by Artistic Licence. The orchestrator sends ArtDMX packets to an Art-Net bridge, which relays them to DMX fixtures. | §2 Walkthrough Step 3b; §12 DMX Profiles. |
| ArtDMX | Art-Net DMX packet | One 512-channel Art-Net data packet. | §2 Walkthrough Step 3b. |
| ArtPoll | Art-Net discovery packet | The Art-Net discovery broadcast used to find bridges. | §2 Walkthrough Step 3a. |
| baking | — | Compiling a timeline into a pre-computed DMX scene stream so playback doesn't need to recompute effects on every frame. | §10 Baking & Playback. |
| battleship search | — | Calibration-discovery strategy that probes a coarse grid across the full pan/tilt range before refining — faster than a dense scan when the beam's reachable region is small. | Appendix B §B.3 Discovery. |
| BFS | Breadth-First Search | Graph-traversal algorithm that explores outward from a seed point one ring at a time. Used in mover calibration to map the visible-region boundary from a first detected beam position. | Appendix B §B.3 Mapping. |
| blink-confirm | — | Reflection-rejection check: after detecting a candidate beam pixel, nudge pan and tilt slightly and verify the detected pixel actually moves. A reflection stays put; a real beam moves. | Appendix B §B.3; issue #658. |
| CLI | Command-Line Interface | A text shell interface (e.g. arduino-cli, gh, PowerShell, bash). The orchestrator's build/flash flow is driven through CLI tools rather than IDEs. | §15 Firmware; CLAUDE.md build commands. |
| CPU | Central Processing Unit | The main processor. CPU0 / CPU1 distinguish the two cores on the ESP32 (WiFi pinned to one, FastLED pinned to the other). | §14 Camera Nodes; CLAUDE.md hardware quirks. |
| CRGB | Color RGB | FastLED's C++ struct for a single RGB pixel. | Firmware modules (GigaLED.h). |
| CRUD | Create, Read, Update, Delete | Shorthand for "all four basic database-style operations." | §4 Fixture Setup; §12 DMX Profiles. |
| CSI | Camera Serial Interface | Raspberry Pi's native ribbon-cable camera port (not supported in SlyLED v1.x — use USB cameras). | §14 Camera Nodes. |
| CV | Computer Vision | Algorithmic image-understanding — the umbrella over solvePnP, RANSAC, beam detection, depth estimation, and YOLO. The auto-tune analyzer evaluator is the CV-only alternative to the AI evaluator. | Appendix A; §14 Camera Nodes; Appendix D. |
| dark reference | — | Snapshot captured with all calibration beams off, subtracted from subsequent frames so beam detection isn't fooled by ambient lighting. | Appendix A §A.5; issue #651. |
| DFU | Device Firmware Update | USB protocol for flashing microcontroller firmware in bootloader mode. The Giga R1 (USB ID 2341:0366) requires the Zadig WinUSB driver installed once before DFU uploads work. | §15 Firmware; CLAUDE.md first-time setup. |
| DHCP | Dynamic Host Configuration Protocol | How devices on a network get an IP address. The board's hostname shows up in DHCP so routers list it by name. | §14 Camera Nodes deployment. |
| DMX | Digital Multiplex | Industry-standard lighting-control protocol — 512 channels per universe, carried over a twisted-pair cable or over Ethernet (Art-Net / sACN). | §2 Walkthrough; §12 DMX Profiles; Appendix B. |
| DNS | Domain Name System | The naming layer that resolves a hostname to an IP address. SlyLED relies on mDNS (zero-config DNS over multicast) for peer-name resolution on the LAN. | §14 Camera Nodes deployment. |
| DOF | Degrees of Freedom | Independent axes a system can move along. SlyLED's parametric mover model has 6 DOF (yaw, pitch, roll, pan offset, tilt offset, plus scale). | Appendix B §B.3 Model fit. |
| EEPROM | Electrically Erasable Programmable Read-Only Memory | Flash-backed non-volatile key/value storage on the D1 Mini (the ESP32 uses NVS instead). Each child board persists its hostname, string config, and per-string GPIO/length there. | §4 Fixture Setup; CLAUDE.md per-board limits. |
| ESP32 | — | Espressif microcontroller family used for LED-performer nodes (WiFi + dual-core, up to 8 LED strings). | §4 Fixture Setup; §15 Firmware. |
| extrinsic | — | A camera's pose (position + rotation) in stage/world space. The solvePnP output. Pair with intrinsic. | Appendix A §A.4. |
| FastLED | — | Arduino library for driving WS2812B-style addressable LED strips. Used on ESP32 and D1 Mini; not reliable on the Giga R1 (custom PWM path instead). | §15 Firmware; CLAUDE.md hardware quirks. |
| fixture | — | Any addressable lighting device — an LED strip, a DMX wash, a moving head, or a camera (which registers as a placeable "fixture" so it has a layout position). | §4 Fixture Setup; throughout. |
| FOV | Field of View | The angular width a camera or lens sees. Stored as fovDeg + fovType (horizontal/vertical/diagonal). Used as an intrinsic fallback when true calibrated intrinsics aren't available. | Appendix A §A.3. |
| FPS | Frames Per Second | Update rate for live playback or emulation. | §11 Show Preview Emulator. |
| FQBN | Fully Qualified Board Name | The arduino-cli identifier for a board target, e.g. arduino:mbed_giga:giga. | §15 Firmware. |
| GET / POST / PUT / DELETE | — | HTTP methods. GET reads, POST creates/triggers, PUT updates, DELETE removes. | §19 API Quick Reference. |
| GPIO | General-Purpose Input/Output | A configurable pin on a microcontroller — used for LED data lines on the ESP32. | §4 Fixture Setup (ESP32 only). |
| homography | — | A 3×3 matrix that maps points on one plane to points on another via projective transform. SlyLED uses a pixel↔floor homography as a fast alternative to full 3D extrinsics during calibration. | Appendix A §A.4. |
| HSV | Hue, Saturation, Value | A colour representation used for colour-filter beam detection (hue bands identify "the green beam" regardless of brightness). | Appendix A §A.8 Beam detection. |
| HTML | HyperText Markup Language | The markup the SPA is built from. | §3 Platform Guide. |
| HTTP | HyperText Transfer Protocol | The request/response protocol behind every /api/* call and every config page served by the orchestrator, performers, and camera nodes. | §19 API Quick Reference; throughout. |
| HUD | Heads-Up Display | An overlay showing live state (used in the 3D viewport). | §5 Stage Layout. |
| ID | Identifier | Any short key that uniquely names something (fixture ID, ArUco marker ID, etc.). | Throughout. |
| IK | Inverse Kinematics | Given a target point, compute the pan/tilt values that aim the beam there. The parametric mover model provides IK once calibration completes. | Appendix B §B.3 Model fit. |
| intrinsic | — | A camera's internal optical parameters: focal length (fx, fy), principal point (cx, cy), and lens distortion. Independent of where the camera is — that's the extrinsic. | Appendix A §A.3. |
| IP | Internet Protocol | Addressing scheme for networked devices (192.168.x.y). | §14 Camera Nodes. |
| JPEG | Joint Photographic Experts Group | Compressed image format used for camera snapshots. | §14 Camera Nodes. |
| JSON | JavaScript Object Notation | The text format used for API request/response bodies and persisted data files. | §19 API Quick Reference. |
| kinematic model | — | Mathematical model that describes how a fixture's motors translate pan/tilt DMX values into an aim direction in stage space. SlyLED fits a 6-DOF kinematic model per calibrated moving head. | Appendix B §B.3. |
| LAN | Local Area Network | The physical/WiFi network the orchestrator and performers share. | §2 Walkthrough. |
| LED | Light-Emitting Diode | Addressable RGB LEDs (WS2812B and similar) are the primary fixture type. | §4 Fixture Setup. |
| LM | Levenberg–Marquardt | Nonlinear least-squares solver used to fit the parametric mover model to calibration samples. | Appendix B §B.3 Model fit. |
| LSQ | Least-Squares | The fitting technique LM refines. When calibration falls back to "median-based" fitting, it's because LSQ wouldn't converge. | Appendix A §A.6. |
| MAC | Media Access Control | A hardware-unique 48-bit identifier baked into every WiFi/Ethernet interface. Performer hostnames are derived from the MAC (e.g. SLYC-XXXX). | §4 Fixture Setup; performer naming. |
| Mbed OS | — | The real-time operating system running on the Arduino Giga R1. Explains why analogWrite() and some libraries behave differently on the Giga. | CLAUDE.md hardware quirks. |
| mDNS | Multicast DNS | Zero-config DNS over multicast — how "SLYC-1234.local" resolves on the LAN without a DNS server. | §14 Camera Nodes deployment. |
| NTP | Network Time Protocol | How performers sync their clocks so runner start times are coordinated. | Protocol (Globals.cpp). |
| NVS | Non-Volatile Storage | ESP32 flash-backed key/value store. SlyLED uses the "slyled" namespace. Equivalent to EEPROM on the D1 Mini. | §4 Fixture Setup. |
| ONNX | Open Neural Network Exchange | Portable neural-network file format. YOLOv8n and Depth-Anything-V2 ship as ONNX files so they run via onnxruntime on ARM. | §14 Camera Nodes. |
| OFL | Open Fixture Library | Community-maintained DMX-fixture profile database. SlyLED can import OFL JSON. | §12 DMX Profiles. |
| orchestrator | — | The desktop (Windows/Mac) or Giga-parent Flask server that hosts the SPA, designs shows, and drives performers and cameras. One of the three tiers. | §1 Getting Started. |
| OS | Operating System | — | CLAUDE.md hardware. |
| OTA | Over-the-Air | Firmware update pushed over WiFi instead of via USB. | §15 Firmware & OTA Updates. |
| PDF | Portable Document Format | The packaged manual format. Generated by tests/build_manual.py. | Appendix C. |
| performer | — | An ESP32, D1 Mini, or Giga-child LED execution node. One of the three tiers. | §1 Getting Started. |
| PnP / solvePnP | Perspective-n-Point | OpenCV algorithm that computes a camera's 3D pose from ≥3 known 2D↔3D point correspondences. SOLVEPNP_SQPNP is the preferred solver; SOLVEPNP_ITERATIVE is the fallback. | Appendix A §A.4. |
| PNG | Portable Network Graphics | Lossless image format used for screenshots. | §2 Walkthrough. |
| PR | Pull Request | Git/GitHub workflow — a proposed change on a branch, reviewed before merge. | Appendix C §C.4. |
| PWM | Pulse-Width Modulation | Dimming technique where the LED is switched on and off fast. On the Giga R1 this is implemented in software because analogWrite() is banned on the onboard RGB pins. | CLAUDE.md hardware quirks. |
| QA | Quality Assurance | Testing role — in SlyLED's workflow, QA runs the Playwright + test suites and files issues rather than patching source. | Appendix C. |
| QR | Quick Response (code) | 2D barcode. Not the same as an ArUco marker — ArUco is designed for solvePnP, QR for data payloads. | — |
| RANSAC | Random Sample Consensus | Robust plane-fitting algorithm — samples random small subsets, finds the model with the most inliers. SlyLED uses it to detect floor and wall planes in noisy point clouds. | Appendix A §A.7. |
| reprojection RMS | — | After solvePnP, project the 3D points back through the solved pose and measure the pixel distance to the detected corners. Reported as root-mean-square across all points. <2 px is excellent, 2–5 px is usable, >5 px means something is wrong. | Appendix A §A.4. |
| REST | Representational State Transfer | An architectural style for HTTP APIs — predictable verbs (GET/POST/PUT/DELETE) acting on noun-shaped URLs. SlyLED's /api/* surface follows REST conventions. | §19 API Quick Reference. |
| RGB / RGBW | Red, Green, Blue [, White] | Standard LED colour models. RGBW adds a dedicated white LED for purer whites. | §4 Fixture Setup. |
| RMS | Root-Mean-Square | Quadratic-mean aggregation of errors (sqrt(mean(x²))). More sensitive to outliers than a plain mean — which is why it's used as a calibration quality metric. | Appendix A §A.4. |
| Rodrigues | — | Mathematical conversion between a rotation vector (rvec from solvePnP) and a 3×3 rotation matrix. cv2.Rodrigues(). | Appendix A §A.4. |
| RSSI | Received Signal Strength Indicator | How strong a WiFi signal a performer is hearing. Reported in dBm; the orchestrator stores it as an unsigned magnitude so "69" means "−69 dBm". | UDP protocol PONG payload. |
| RTOS | Real-Time Operating System | An OS with deterministic timing guarantees. Mbed OS on the Giga is an RTOS. | CLAUDE.md hardware. |
| runner | — | A step sequencer loaded into a performer. Each step is an action (colour, pattern, LED range) with a duration; the runner loops the step list in sync with the orchestrator. | §4 Fixture Setup; §13 Preset Shows. |
| RX / RY / RZ | — | Rotations about the X, Y, Z axes of the stage frame, in degrees. In schema v2: rx = pitch, ry = roll, rz = yaw/pan. Never read rotation[1] or rotation[2] directly — always go through rotation_from_layout(). | Appendix A §A.9. |
| sACN | Streaming ACN | DMX-over-Ethernet alternative to Art-Net, defined by RFC 7724. SlyLED speaks both. | §12 DMX Profiles. |
| SCP | Secure Copy Protocol | File transfer over SSH. How camera firmware reaches the Orange Pi / Raspberry Pi. | §15 Firmware → Camera deploy. |
| solvePnP | — | See PnP. | Appendix A §A.4. |
| SPA | Single-Page Application | The desktop orchestrator UI is one HTML page that loads JavaScript modules instead of navigating between pages. | §3 Platform Guide. |
| SQPNP | — | A specific solvePnP algorithm variant (cv2.SOLVEPNP_SQPNP) chosen because it tolerates fewer correspondences than the iterative solver. | Appendix A §A.4. |
| SRAM | Static Random-Access Memory | The fast, volatile RAM on a microcontroller. Tight budget on the D1 Mini — the manual warns against String objects and heap allocation. | CLAUDE.md performance rules. |
| SSH | Secure Shell | Encrypted remote-login protocol. How the orchestrator reaches camera-node shells for firmware deployment. | §15 Firmware → Camera deploy. |
| SVG | Scalable Vector Graphics | Vector image format used by diagram exporters. | Appendix C. |
| TCP | Transmission Control Protocol | Reliable, connection-based networking. HTTP traffic (config pages, API calls) rides on TCP. | UDP protocol discussion. |
| tiling | — | Sliced-Aided Hyper-Inference (SAHI)-style detection: break a large image into overlapping patches, run the detector on each, stitch results. Improves small-object detection accuracy at the cost of runtime. Controlled by the tile option on /scan. | §14 Camera Nodes. |
| TTL | Time-To-Live | A timeout after which a resource (e.g. a mover claim) auto-expires. Mover-control claims have a 15 s TTL. | Appendix B §B.7. |
| UDP | User Datagram Protocol | Connectionless, best-effort networking. Used for all orchestrator↔performer traffic (discovery, actions, runner control) because it's low-latency and the wire protocol tolerates occasional packet loss. | Wire protocol; CLAUDE.md §UDP binary protocol. |
| UI | User Interface | — | Throughout. |
| universe | — | A DMX addressing space — 1–512 channels. A show typically spans multiple universes; Art-Net addresses them as net.subnet.universe. | §12 DMX Profiles. |
| URL | Uniform Resource Locator | Web address. | §14 Camera Nodes. |
| USB | Universal Serial Bus | — | §15 Firmware USB flash. |
| V4L2 | Video for Linux 2 | The kernel video-capture API used by camera nodes (cv2.VideoCapture on Orange Pi / Raspberry Pi). SoC ISP video nodes like sunxi-vin and bcm2835-isp are filtered out — only regular USB cameras register. | §14 Camera Nodes. |
| WiFi | — | 802.11 wireless networking. Performers and camera nodes join the orchestrator's LAN over WiFi. | §2 Walkthrough. |
| WLED | — | Popular open-source firmware for ESP32/8266-based LED controllers. SlyLED includes a bridge so WLED devices can appear as performers. | §4 Fixture Setup; desktop/shared/wled_bridge.py. |
| WS2812B | — | Common addressable-RGB LED chip (aka "NeoPixel"). The ESP32 RMT peripheral drives it in hardware; the D1 Mini bit-bangs it in software. | §4 Fixture Setup. |
| YOLO | You Only Look Once | Single-pass object-detection neural network. SlyLED camera nodes run YOLOv8n via ONNX Runtime for person/object detection on POST /scan. | §14 Camera Nodes. |
| ZIP | — | Archive file format, used for the release bundle. | §15 Firmware Registry. |

Not sure what something means? If a term appears in the manual but isn't in this table, that's a bug in the glossary — open an issue or PR against #663.

────────────────────────────────────────

## Appendix A — Camera Calibration Pipeline (DRAFT)

⚠ DRAFT — assumes all in-flight work is merged. This appendix describes the camera-calibration pipeline under the assumption that issues #610, #651–#661, and #357 are fully implemented. Some features documented below are partially merged today (notably full intrinsic calibration of every camera, dark-reference integration into the mover pipeline per #651, and the floor-view polygon target filter per #659). See docs/DOCS_MAINTENANCE.md for the current merge status and the criteria for removing this banner. Issue #662.

Camera calibration runs as a one-time setup per camera node and must be repeated whenever a camera is physically moved or re-aimed. It produces, per camera: an intrinsic matrix K (focal length + principal point + distortion), an extrinsic pose (stage-space position + rotation), and — for stages that will run point-cloud scans — a depth-anchor fit that corrects monocular depth to stage metric.

### A.1 Pipeline overview

[Mermaid diagram — see docs/diagrams/ for rendered source]

%%{init: {'theme':'neutral'}}%%
flowchart TD
    Start([Register camera node]) --> Deploy[Deploy firmware via SSH+SCP]
    Deploy --> Intrinsic{1 — Intrinsic calibration}
    Intrinsic -->|Checkerboard path| CBCap[Capture 15-30 checkerboard frames<br/>~500ms/frame]
    Intrinsic -->|ArUco path| ArCap[Capture 5-10 ArUco snapshots]
    CBCap --> CBCompute[cv2.calibrateCamera<br/>~2s for 3 frames]
    ArCap --> ArCompute[cv2.calibrateCamera<br/>a few seconds]
    CBCompute --> Saved[Save intrinsic_camN.json on camera]
    ArCompute --> Saved
    Intrinsic -.->|skip| FOV[Fall back to FOV-derived K<br/>intrinsic_source = fov-estimate]

    Saved --> Survey[2 — ArUco marker survey]
    FOV --> Survey
    Survey --> MarkerReg[POST /api/aruco/markers<br/>id, size, x/y/z, rx/ry/rz, label]
    MarkerReg --> Coverage[GET /api/aruco/markers/coverage]
    Coverage --> Enough{>= 3 markers<br/>visible per camera?}
    Enough -->|no| AddMarker[Add marker at<br/>recommendation pin]
    AddMarker --> Coverage
    Enough -->|yes| DarkRef[3 — Dark-reference capture<br/>~100ms/cam]

    DarkRef --> StageMap[4 — Extrinsic solve<br/>~5-10s multi-snapshot]
    StageMap --> PnP[cv2.solvePnP SQPNP<br/>best-per-ID corners]
    PnP --> StoreExt[Store rotation rx/ry/rz + position<br/>rotationSchemaVersion: 2]

    StoreExt --> Scan[5 — Optional: space scan]
    Scan --> PointCloud[/point-cloud per cam<br/>~6.5s/cam metric model]
    PointCloud --> Anchor[Depth anchor fit<br/>scale + offset]
    Anchor --> Merge[space_mapper<br/>cross-cam filter + floor normalize]
    Merge --> Surfaces[surface_analyzer RANSAC<br/>floor + walls + obstacles]
    Surfaces --> Done([Stage geometry ready])

    Scan -.->|skip| Done
    StageMap -.->|RMS > 5px or too few markers| Error[Calibration error]
    PnP -.->|no convergence| Error
    Error --> Retry[Add markers or increase snapshots]
    Retry --> Coverage

### A.2 ArUco marker surveying

Physical ArUco markers mounted on the stage floor, walls, or rigging define the ground-truth geometry that every subsequent camera-calibration step references.

Registry schema — each marker is stored with:

| Field | Type | Notes |
| --- | --- | --- |
| id | int (0–49) | ID within the CV2_DICT_4X4_50 dictionary |
| size | float mm (≥1) | Physical edge length; default 100 mm |
| x, y, z | float mm | Stage-space position; usually z=0 for floor markers |
| rx, ry, rz | float deg | Marker orientation — see §A.9 for axis convention |
| label | string (≤60 chars) | Operator annotation, e.g. north-entrance |

Endpoints

| Method | Path | Purpose |
| --- | --- | --- |
| GET | /api/aruco/markers | Full registry (dictId, markers[]) |
| POST | /api/aruco/markers | Upsert one or more markers by ID |
| DELETE | /api/aruco/markers/<id> | Remove a marker by ID |
| GET | /api/aruco/markers/coverage | Per-camera visible-ID report with a placement recommendation |

Coverage pre-flight — before starting the extrinsic solve, run GET /api/aruco/markers/coverage. The response lists which IDs each camera currently sees, the stage-space hull covered by the registered markers, and a recommendation object indicating which camera has the weakest coverage and where the operator should place the next marker.

Expected timing — marker registration is instant (JSON write). Coverage check takes one snapshot per registered camera, typically 50–200 ms per camera.

Fallback — none. Without surveyed markers there is no extrinsic solve; the camera defaults to identity rotation and (0, 0, 0) position.

### A.3 Intrinsic calibration

Intrinsic calibration produces the camera's K matrix (fx, fy, cx, cy) and distortion coefficients. It is a per-lens, per-resolution, one-time step. Two independent paths are supported:

Path A — Checkerboard (camera-side)

| Method | Path | Purpose |
| --- | --- | --- |
| POST | /calibrate/intrinsic/capture | Grab one frame, find ≤10 checkerboards (4×9, 25 mm squares default), accumulate corners |
| POST | /calibrate/intrinsic/compute | Run cv2.calibrateCamera on accumulated frames; save to /opt/slyled/calib/intrinsic_camN.json |
| GET | /calibrate/intrinsic?cam=N | Retrieve the saved calibration |
| DELETE | /calibrate/intrinsic | Remove saved calibration |
| POST | /calibrate/intrinsic/reset | Clear accumulated frames (does not touch saved file) |

Expected timing — ~500 ms per frame capture, 2–5 s compute with the minimum three frames. Targets for a usable calibration: 15–30 frames, RMS < 0.3 pixels.

Path B — ArUco (orchestrator-side)

| Method | Path | Purpose |
| --- | --- | --- |
| POST | /api/cameras/<fid>/aruco/capture | Snapshot + ArUco detection, accumulate per-fixture corners |
| POST | /api/cameras/<fid>/aruco/compute | Pool corners across frames, cv2.calibrateCamera, POST result to camera node for persistence |
| POST | /api/cameras/<fid>/aruco/reset | Clear accumulated frames |
| GET | /api/cameras/<fid>/intrinsic | Proxy GET to the camera node |
| DELETE | /api/cameras/<fid>/intrinsic | Proxy DELETE to the camera node |
| POST | /api/cameras/<fid>/intrinsic/reset | Proxy POST to the camera node |

Expected timing — 5–10 captures × a few hundred ms each, compute in a few seconds.

Fallback — if no calibration file is available, the orchestrator falls back to an FOV-derived K on the fly: fx = (w/2) / tan(h_fov/2), fy = fx, cx = w/2, cy = h/2, distortion zero. The /stage-map response reports intrinsic_source: "fov-estimate" when this path is used. Accuracy drops to roughly ±15% of true focal length.

Persistence — saved to the camera node at /opt/slyled/calib/intrinsic_camN.json; survives reboots.

### A.4 Extrinsic solve (solvePnP)

Given surveyed markers in stage space and their detected pixel corners, the orchestrator computes each camera's pose via cv2.solvePnP (see PnP, RANSAC is not used here — PnP is a direct algebraic solve, not a consensus method).

[Mermaid diagram — see docs/diagrams/ for rendered source]

%%{init: {'theme':'neutral'}}%%
sequenceDiagram
    actor Op as Operator
    participant SPA as SPA (calibration.js)
    participant Orch as Orchestrator
    participant Cam as Camera node

    Op->>SPA: Click Solve
    SPA->>Orch: POST /api/cameras/fid/stage-map

    Note over Orch,Cam: Multi-snapshot aggregation
    loop maxSnapshots (default 6)
        Orch->>Cam: GET /snapshot?cam=N
        Cam-->>Orch: JPEG
        Orch->>Orch: detectMarkers + keep best per ID
    end

    Orch->>Cam: GET /calibrate/intrinsic?cam=N
    Cam-->>Orch: K or 404
    alt K available
        Orch->>Orch: intrinsic_source=calibrated
    else fallback
        Orch->>Orch: intrinsic_source=fov-estimate
    end

    Orch->>Orch: cv2.solvePnP SQPNP
    alt convergence
        Orch->>Orch: projectPoints RMS check
        Orch->>Orch: Rodrigues -> tilt/pan/roll
        Orch-->>SPA: ok, reprojectionRmsPx
    else RMS > 5 or no convergence
        Orch->>Orch: try SOLVEPNP_ITERATIVE
        alt still fails
            Orch-->>SPA: error
        end
    end

Endpoint — POST /api/cameras/<fid>/stage-map with {cam, markers, markerSize, maxSnapshots}.

Preconditions — ≥3 surveyed markers registered (or ≥2 if all floor-coplanar), each visible in at least one snapshot. Multi-snapshot aggregation handles frame-to-frame transients; six snapshots is a good default.

Algorithm — for each snapshot, the orchestrator detects markers and keeps the single detection with the largest perimeter per ID (largest = closest = best sub-pixel corners). Correspondences from each detected corner to the surveyed 3D point feed into cv2.solvePnP(..., flags=SOLVEPNP_SQPNP), with SOLVEPNP_ITERATIVE as a fallback solver.

Expected timing — one multi-snapshot run: 5–10 s (dominated by snapshot capture).

Output — tvec (stage mm) → camera position; rvec → Rodrigues → tilt/pan/roll → stored in camera.rotation using schema v2 (§A.9). Also reports reprojectionRmsPx.

Fallbacks

| Failure | Behaviour | Operator action |
| --- | --- | --- |
| Fewer than 3 markers matched across all snapshots | Error; pose not updated | Add markers or reposition camera |
| SQPNP does not converge | Retry with SOLVEPNP_ITERATIVE | None automatic |
| Reprojection RMS > 5 px | Pose stored but flagged | Verify surveyed marker positions and markerSize; recapture intrinsics |
| No intrinsics on camera | Fall back to FOV-derived K, report intrinsic_source: "fov-estimate" | Run §A.3 for better accuracy |

### A.5 Dark-reference capture (#651)

A dark-reference frame is a snapshot taken with all calibration beams off so that beam detection in subsequent calibration steps can subtract ambient lighting.

Endpoint — POST /dark-reference on the camera node, body {cam: -1} (all cameras) or {cam: N}.

Behaviour — captures one frame per camera, stores in the BeamDetector in-memory buffer (not persisted across reboots).

Expected timing — ~100 ms per camera (V4L2 frame grab).

When it runs — automatically at the start of each mover calibration run (see Appendix B §B.3). Can also be triggered manually before running beam-detect calls.

Fallback — if the beam detector module is not available on the node, the endpoint returns 503 and the caller proceeds without dark-reference subtraction. Beam detection still works but is more sensitive to ambient light.

### A.6 Point cloud + multi-camera merge

Point-cloud generation produces a 3D representation of the stage from monocular depth plus camera pose. It is optional — only needed for features that reason about stage surfaces (mover-calibration target filtering, tracking, spatial effects on arbitrary geometry).

Camera-side endpoint — POST /point-cloud with {cam, maxPoints, maxDepthMm}. Returns {points: [[x,y,z,r,g,b], ...], pointCount, inferenceMs, calibrated, fovDeg}.

Depth models

| Model | File | Output | Typical inference on ARM |
| --- | --- | --- | --- |
| Metric (Depth-Anything-V2 Metric Indoor Small) | /opt/slyled/models/depth_anything_v2_metric_indoor.onnx | Depth in mm directly | ~6.5 s / frame |
| Disparity (Depth-Anything-V2 Small, fallback) | /opt/slyled/models/depth_anything_v2_small.onnx | Normalized [0,1]; caller scales by maxDepthMm | ~6.5 s / frame |

The camera selects the metric model by default; the disparity model is a fallback when the metric file is absent. Preference file: /opt/slyled/models/active_depth_model.

Multi-camera merge — POST /api/space/scan on the orchestrator runs, in order:

Fetch per-camera point clouds (try /point-cloud on the node; orchestrator-side depth if camera path unavailable).

Per-camera depth anchor (#581) — two-parameter scale + offset least-squares fit so monocular depth agrees with stage geometry. Reject outliers >2σ, refit. If RMS > 2000 mm, fall back to a median-based coarse fit.

Transform to stage coordinates using the camera's stored rotation + position.

Cross-camera consistency filter (#582) — if ≥2 cameras are present, reject points that appear in only one camera's view where another camera should have seen them (filters monocular hallucinations).

Floor normalization — compute 5th-percentile Z per camera, shift cloud so the average floor lands at Z=0; fall back to RANSAC floor detection if the camera-anchored method fails.

Z-marker alignment (#599) — if any registered marker has z < 50 mm and zero rotation, treat it as ground truth and shift the cloud so those markers sit at Z=0.

Expected timing — 30–60 s end-to-end for a typical two-camera setup; dominated by per-camera depth inference.

Anchor quality classification in the response (depthAnchor.quality): "ok" (RMS ≤ 500 mm), "degraded" (500–2000 mm), "fallback" (>2000 mm, median fit used). Degraded or fallback quality means downstream features (surface fitting, tracking) may be inaccurate.

### A.7 Surface analysis (RANSAC)

After a merged point cloud is available, desktop/shared/surface_analyzer.py extracts structural surfaces so the orchestrator can reason about them (obstacle-aware target picking, beam-surface intersection for mover calibration).

| Surface | Algorithm | Expected runtime | Failure mode |
| --- | --- | --- | --- |
| Floor | RANSAC up to 200 trials, sample 3 points, plane fit, require vertical normal (dot-z > 0.95), ≥5% inlier share | 100–500 ms | Too few inliers → no floor reported |
| Walls | RANSAC on non-floor points, 2-point vertical-plane fit, ≥50 inliers or ≥5%, max 4 walls | 500 ms–2 s total | Silent; fewer walls reported |
| Obstacles | 300 mm XY grid + flood-fill, cluster size ≥20 points, classify as pillar if tall+thin else obstacle | 100–500 ms | Silent; sparse clusters rejected |

Ray-cast intersection — beam_surface_check() answers "which surface does a beam ray hit first?" Used by mover calibration to interpret detected-beam pixels as stage points when the beam lands on a wall or pillar rather than the floor (see #585/#260). Typical runtime 10–50 ms.

### A.8 Beam detection (interface used by mover calibration)

Beam detection is documented here because it is how the camera pipeline feeds moving-head calibration (Appendix B). The endpoints live on the camera node.

| Method | Path | Purpose | Typical runtime |
| --- | --- | --- | --- |
| POST | /beam-detect | Single-frame detection; color filter + brightness + saturation + compactness | <100 ms |
| POST | /beam-detect/flash | Capture ON frame → wait offDelayMs → capture OFF → diff; immune to ambient shifts | <100 ms + offDelayMs |
| POST | /beam-detect/center | Multi-beam fixture: detect N beams, return cluster centre | <150 ms |

Color filtering uses HSV hue ranges (beam_detector.py): red [0,60] ∪ [168,180], green [35,85], blue [100,130], magenta [140,170]; white falls back to brightness only.

Validation checks per contour: mean-V ≥ 160 (brightness); mean-S ≥ 80 for colored beams (saturation); aspect ≤ 5 (compactness).

### A.9 Rotation schema v2 (axis convention)

Camera and DMX fixture rotations share a unified convention (issues #586, #600). fixture.rotation is a list [rx, ry, rz] in degrees, axis-letter-matched to the Z-up stage frame:

[Mermaid diagram — see docs/diagrams/ for rendered source]

%%{init: {'theme':'neutral'}}%%
flowchart LR
    subgraph Stage["Stage coordinate frame (Z-up)"]
        X["+X — stage-left"]
        Y["+Y — downstage forward"]
        Z["+Z — up"]
    end

    subgraph Rotation["rotation = [rx, ry, rz] (deg)"]
        RX["rx = pitch<br/>about X<br/>rx > 0 aims DOWN"]
        RY["ry = roll<br/>about Y (stage-forward)<br/>ry > 0 rotates image clockwise<br/>(viewed from behind the camera)"]
        RZ["rz = yaw / pan<br/>about Z (stage-up)<br/>rz > 0 aims toward +X"]
    end

    Stage --> Rotation

Canonical read path — always route through:

Python: desktop/shared/camera_math.py::rotation_from_layout(rot) → (tilt, pan, roll), then build_camera_to_stage(tilt, pan, roll) for the 3×3 matrix.

SPA: rotationFromLayout(rot) in spa/js/app.js.

Never read `rotation[1]` or `rotation[2]` directly — those indices swap between v1 and v2 files.

Schema migration — imported project files carrying layout.rotationSchemaVersion < 2 (or missing) have ry and rz swapped on load (v1 used ry=pan, rz=roll; v2 uses ry=roll, rz=pan). Current exports always write rotationSchemaVersion: 2.

### A.10 Failure modes & operator expectations

| Phase | Symptom | Probable cause | Operator action |
| --- | --- | --- | --- |
| Marker survey | Coverage recommendation persists after adding markers | Marker outside every camera's FOV | Move marker toward the recommended pin, or reposition camera |
| Intrinsic capture | "Board not found" | Lighting too low, angle too oblique, printout wavy | Flatten print, reposition, add light |
| Intrinsic compute | RMS > 1.0 px | Too few frames, poor angle variety | Capture 10+ more frames from diverse angles |
| Extrinsic solve | markersMatched < 3 | Markers not visible in any snapshot | Increase maxSnapshots, reposition camera, add markers |
| Extrinsic solve | reprojectionRmsPx > 5 | Bad intrinsics or wrong markerSize | Run §A.3; verify physical marker size matches registry |
| Extrinsic solve | intrinsic_source: fov-estimate | Camera has no saved intrinsic calibration | Optional: run §A.3 for ±2–5 px accuracy instead of ±10–20 px |
| Dark reference | 503 response | Beam detector module missing on camera | Redeploy firmware from the Firmware tab |
| Depth estimate | "model unavailable" | ONNX file not deployed | Deploy via Firmware → Camera tab |
| Anchor fit | quality: fallback | Monocular depth disagrees strongly with stage geometry | Verify stage bounds and camera pose; may be a solvePnP error upstream |
| Surface analysis | No floor reported | Cloud too sparse or noisy, camera aimed too high | Add cameras, aim lower, verify depth model |

### A.11 File locations & persistence

| Data | Location | Format | Notes |
| --- | --- | --- | --- |
| ArUco registry | desktop/shared/data/aruco_markers.json | JSON list | Persisted on every marker POST/DELETE |
| Camera fixtures | desktop/shared/data/fixtures.json | JSON list | Includes stored pose + rotation |
| Layout positions | desktop/shared/data/layout.json | JSON | Must carry rotationSchemaVersion: 2 |
| Intrinsic calibration (camera-side) | /opt/slyled/calib/intrinsic_camN.json | JSON | fx, fy, cx, cy, distCoeffs, imageSize, rmsError, frameCount |
| Point cloud cache | desktop/shared/data/pointcloud.json | JSON | From last /api/space/scan run |

────────────────────────────────────────

## Appendix B — Moving-Head Calibration Pipeline

Updated for orchestrator v1.7.83. The pipeline described here is
the post-#784 architecture: a single canonical aim-vector
(aim_stage, #806) is the source of truth for every head, the
ParametricFixtureModel + Levenberg–Marquardt fit (#488) is the
primary IK, and the legacy SMART pipeline that powered earlier
calibration releases has been retired. The phase-by-phase content
below has been ground-truthed against the current
mover_calibrator.py and parametric_mover.py.

Moving-head calibration runs per DMX moving-head fixture after the camera(s) covering its reachable region have been calibrated (Appendix A). It produces a sample set + parametric 6-DOF kinematic model that lets the orchestrator translate stage-space targets into exact pan/tilt DMX values, enabling IK (inverse kinematics) for the Track action and spatial effects.

### B.1 Pipeline overview

[Mermaid diagram — see docs/diagrams/ for rendered source]

%%{init: {'theme':'neutral'}}%%
flowchart TD
    Start([Start Calibration]) --> Claim[Claim mover<br/>15s TTL]
    Claim --> Warmup[1 — Warmup<br/>~30s]
    Warmup --> Discovery{2 — Discovery<br/>45-55s}
    Discovery -->|Battleship path| BS[Battleship 4x4 grid<br/>+ blink-confirm nudge<br/>~10-15s]
    Discovery -->|Legacy path| Coarse[Coarse 10x7 scan<br/>+ fine spiral fallback<br/>~45-55s]
    BS --> MapConv[3 — Mapping / Convergence<br/>35-50s]
    Coarse --> MapConv
    MapConv --> Grid[4 — Grid build<br/>&lt;1s]
    Grid --> Verify[5 — Verification sweep<br/>3-5s advisory]
    Verify --> Fit[6 — LM model fit<br/>verify_signs → single-combo LM<br/>&lt;1s]
    Fit --> HoldOut{7 — Held-out<br/>parametric gate<br/>#654}
    HoldOut -->|pass| Save[8 — Save +<br/>moverCalibrated=true]
    HoldOut -->|fail| OperatorRetry[Accept / Retry prompt]
    OperatorRetry -->|Retry| Discovery
    OperatorRetry -->|Accept| Save
    Save --> Release[Release claim + blackout]
    Release --> Done([Complete])

    Discovery -.->|no beam after 80 probes| Error
    MapConv -.->|fewer than 6 samples| Error
    Fit -.->|no sign combo fits| Error
    Error[Phase error] --> Blackout[_cal_blackout<br/>512 zeros + release claim]

### B.2 Phase reference

| # | Phase | Status string | Typical duration | Progress % | Fallback on failure |
| --- | --- | --- | --- | --- | --- |
| 1 | Warmup | warmup | 30 s (configurable via warmupSeconds) | 2–8 | Log warning, continue without warmup |
| 2 | Discovery (legacy) | discovery | 45–55 s | 10–30 | Abort with error status after 80 probes |
| 2′ | Discovery (battleship) | battleship → confirming | 10–15 s | 10–25 | Abort with error; can fall back to legacy discovery |
| 3 | Mapping (legacy BFS) | mapping | 35–50 s | 35–70 | Abort if <6 samples |
| 3′ | Convergence (v2) | sampling | 30–60 s (N targets × ~1 s each) | 30–70 | Abort if convergence fails on multiple targets |
| 4 | Grid build | grid | <1 s | ~80 | Abort if sample spread insufficient |
| 5 | Verification sweep | verification | 3–5 s (3 held-out points) | ~90 | Advisory only — does not block save |
| 6 | Model fit (LM) | fitting | <1 s | 85–95 | verify_signs first → single-combo LM; full four-combo fallback if a sign probe misses |
| 7 | Held-out parametric gate (#654) | holdout | 2–5 s (N unseen targets) | 95–98 | Surface Accept/Retry prompt |
| 8 | Save | complete | <1 s | 100 | Write error logged but does not affect moverCalibrated flag |

Additional top-level statuses: cancelled, error, done.

[Mermaid diagram — see docs/diagrams/ for rendered source]

%%{init: {'theme':'neutral'}}%%
stateDiagram-v2
    [*] --> Idle
    Idle --> Claimed: POST /claim
    Claimed --> Warmup: start
    Warmup --> Discovery
    Discovery --> Confirming: candidate found<br/>(battleship path)
    Confirming --> Discovery: nudge fails<br/>(#658 reflection)
    Confirming --> Mapping: nudge confirms
    Discovery --> Mapping: beam found<br/>(legacy path)
    Mapping --> Grid: samples >= 6
    Grid --> Verification
    Verification --> Fitting: advisory pass or skip
    Fitting --> HeldOut: LM converged
    HeldOut --> Complete: within tolerance
    HeldOut --> Idle: retry (operator)
    Complete --> Idle: released

    Discovery --> Error: 80 probes exhausted
    Mapping --> Error: insufficient spread
    Fitting --> Error: no sign combo fits
    Error --> Blackout
    Warmup --> Cancelled: operator cancel
    Discovery --> Cancelled: operator cancel
    Mapping --> Cancelled: operator cancel
    Confirming --> Cancelled: operator cancel
    Cancelled --> Blackout

    Warmup --> TimedOut: phase budget #653
    Discovery --> TimedOut: phase budget #653
    Mapping --> TimedOut: phase budget #653
    TimedOut --> Blackout

    Blackout --> Idle: claim released

### B.3 Phase-by-phase detail

[Mermaid diagram — see docs/diagrams/ for rendered source]

%%{init: {'theme':'neutral'}}%%
sequenceDiagram
    actor Op as Operator
    participant SPA as SPA (calibration.js)
    participant Orch as Orchestrator<br/>(mover_calibrator.py)
    participant DMX as Art-Net bridge
    participant Cam as Camera node

    Op->>SPA: Click Start Calibration
    SPA->>Orch: POST /api/calibration/mover/fid/start
    Orch->>Orch: _set_calibrating(fid, True)

    Note over Orch,DMX: Warmup (~30s)
    loop warmup sweep steps
        Orch->>DMX: _hold_dmx(pan,tilt,colour)
    end

    Note over Orch,Cam: Discovery (~45-55s or ~10-15s battleship)
    loop battleship grid or coarse 10x7
        Orch->>DMX: set pan,tilt
        Orch->>Cam: POST /beam-detect
        Cam-->>Orch: pixel or null
    end
    opt battleship hit — blink-confirm (#658)
        Orch->>DMX: pan + nudge
        Orch->>Cam: /beam-detect
        Orch->>DMX: tilt + nudge
        Orch->>Cam: /beam-detect
    end

    Note over Orch,Cam: Mapping / Convergence (~35-50s)
    loop BFS or per-target converge
        Orch->>DMX: set pan,tilt
        Orch->>Cam: /beam-detect (dual-capture #655)
    end

    Note over Orch: Grid build (<1s, sync)
    Note over Orch,Cam: Verification sweep (3-5s)
    Note over Orch: verify_signs probes → single-combo LM fit
    Note over Orch,Cam: Held-out parametric gate (#654)

    alt within tolerance
        Orch->>Orch: fixture.moverCalibrated=true
    else too high
        Orch-->>SPA: prompt Accept/Retry
    end

    Orch->>DMX: blackout
    Orch->>Orch: _set_calibrating(fid, False)
    SPA-->>Op: complete

#### 1. Warmup

Purpose — cycle the fixture through full pan/tilt range so motor belts are thermally and mechanically settled before measurements start. Reduces backlash artifacts in early samples.

Preconditions — valid DMX profile on fixture; Art-Net engine running; calibration lock engaged.

Expected duration — 30 s by default (warmupSeconds parameter). Six sub-sweeps (pan±, tilt±, two diagonals) × 20 steps each, ~0.25 s per step.

Operator expectation — beam sweeps visibly across the stage; progress bar creeps from 2% to 8%.

Fallback — if warmup raises an exception, log warning and skip; calibration continues.

Cancel — _check_cancel() inside each _hold_dmx loop raises CalibrationAborted.

#### 2. Discovery

Two code paths exist. The battleship path is preferred when camera homography is reliable; legacy path is the fallback when homography is unavailable (e.g. no surveyed markers visible).

Battleship (preferred, `battleship` + `confirming` status):

Coarse 4×4 grid at pan/tilt bin centres {0.125, 0.375, 0.625, 0.875}² — 16 probes. Per #661, grid density scales with pan range and expected beam width; defaults to 4×4 but can reduce to 3×3 or expand to 5×5 for wide-pan fixtures.

On the first detected pixel candidate, run the blink-confirm routine (#658): nudge pan by confirm_nudge_delta (≈ 0.02 of full range) and verify the detected pixel moves; nudge tilt likewise. If both pixel deltas exceed min_delta, the candidate is confirmed. Otherwise it was a reflection and is rejected — discovery resumes.

Expected duration 10–15 s (16 probes × 0.6 s settle, plus 4 nudges × 0.6 s when a candidate hits).

Fallback on failure — fall through to the legacy coarse+spiral path, or abort to error.

Legacy (`discovery` status):

Initial probe at the warmstart aim (model prediction or geometric estimate from camera FOV).

Coarse 10×7 grid: pan bins 0.02 + 0.96·i/9, tilt bins 0.1 + 0.85·j/6 — 70 probes. Each probe goes through _wait_settled, which reuses the same adaptive-settle machinery (SETTLE_BASE = 0.4 s with [0.4, 0.8, 1.5] s escalation) used in the mapping phase — not a separate fixed SETTLE constant.

If the coarse sweep misses, spiral outward from the warmstart aim in rectangular shells at STEP = 0.05, up to MAX_PROBES = 80 total.

Expected duration — 45–55 s worst-case.

Fallback on failure — abort with error; call _cal_blackout().

Operator expectations — beam sweeps through a visible grid of positions. If the beam is clearly landing where the camera can see it but detection fails, check §A.5 dark-reference and §A.8 color filter config.

#### 3. Mapping (legacy BFS) / Convergence (v2)

Legacy BFS (`mapping` status):

BFS from the discovered (pan, tilt, pixel) seed. Each step detects the beam and, on success, enqueues four neighbours (up/down/left/right by STEP). Beam loss marks the current cell as a visible-region boundary; stale detections (where the pixel barely moves despite a large pan/tilt delta) are rejected as noise.

Adaptive settle (#655) scales per-probe settle time by movement distance, with escalation levels [0.4, 0.8, 1.5] s. Dual-capture with a 0.2 s verify gap and a 30-pixel drift threshold filters mid-move frames; median filtering across the capture pair rejects outliers.

Targets — _map_target = 50 samples (hard minimum 6; bounded by MAX_SAMPLES = 80).

Expected duration — 35–50 s.

v2 Convergence (`sampling` status):

For each target from pick_calibration_targets (filtered through the camera floor-view polygon per #659), converge the beam on the target pixel via converge_on_target_pixel.

Bracket-and-retry refine (#660) — initial bracket_step = 0.08. When the beam is lost, halve the step and walk back toward the best-known-good offset in the error direction. BRACKET_FLOOR is fixture-dependent: 1 / (2^pan_bits − 1) — about 0.0039 on 8-bit pan and 0.0000153 on 16-bit, so the loop exhausts to the fixture's actual DMX resolution instead of the old 8-bit-only 1/255 floor (#679). Reset bracket_step to 0.08 on beam re-acquisition. Typical convergence: 5–10 iterations; max 25.

Expected duration — 30–60 s (N targets × ~1 s each).

Fallback — if fewer than 6 samples are collected, abort with error.

#### 4. Grid build

Pure compute: extract unique pan/tilt values from samples, sort, nearest-neighbour fill for missing cells.

Expected duration — <100 ms, no I/O.

Fallback — if sample spread insufficient to form a grid, abort with error.

#### 5. Verification sweep

Pick 3 random targets inside the grid bounds, avoiding fit samples by ≥0.05 pan/tilt margin, with a 10% interior shrink to dodge weak-interpolation edges.

For each: predict pixel via grid lookup, detect actual beam, compute pixel error.

Expected duration — 3–5 s (3 × ~1 s settle+detect).

Advisory only — logs a warning if any point fails; does not block the save.

#### 6. Model fit (parametric, Levenberg-Marquardt)

Sign verification (#652 / §8.1) runs FIRST, before the LM fit. Right after discovery returns (pan, tilt, pixel), the thread issues two extra probes (pan + 0.02, tilt + 0.02) and calls verify_signs on the pixel deltas to compute (pan_sign, tilt_sign). Those signs flow into fit_model(..., force_signs=(ps, ts)) so the LM runs one solve instead of four.

Fallback — if any sign probe fails to detect a beam, force_signs is left None and fit_model runs the full four-combo search and picks the lowest-RMS candidate.

When the top two candidates agree to within 0.2° RMS AND the caller didn't supply force_signs, FitQuality.mirror_ambiguity is set True and surfaced on the status endpoint (#679) so the UI can flag the calibration for a manual re-run.

scipy.optimize.least_squares with soft_l1 loss (f_scale=0.05) fits five continuous parameters (mount yaw/pitch/roll + pan/tilt offsets); up to 120 iterations.

Expected duration — <1 s total (verify_signs + single LM solve).

Fallback — if all 4 combos fail to converge, raise RuntimeError; caller aborts with error.

#### 7. Held-out parametric gate (#654)

After fit, drive the fixture to 2–3 unseen targets (not used in discovery, mapping, or verification) and measure pixel-level residual against model prediction.

If residual is within tolerance, set fixture["moverCalibrated"] = True.

If residual exceeds tolerance, return the result to the SPA as an Accept/Retry prompt: operator may accept (flag still set, marked as degraded) or retry calibration from discovery.

Expected duration — 2–5 s.

#### 8. Save + release

Persist samples, model dict, fitQuality metrics, and per-phase metadata to desktop/shared/data/fixtures.json.

Set fixture["moverCalibrated"] = True (if not already).

Release the calibration lock via _set_calibrating(fid, False) — the mover-follow engine resumes writing pan/tilt.

Blackout the fixture.

### B.4 Time budget + blackout-on-timeout (#653)

Each phase has a per-phase wall-clock budget. If exceeded, the phase raises PhaseTimeout, caught by the top-level calibration thread, which then:

Calls _cal_blackout() — sends 512 zeros to the fixture's universe for 0.3 s.

Releases the calibration lock.

Sets job["status"] = "error", job["phase"] = "<phase>_timeout".

Flags tier-2 handoff in the status dict so the SPA can suggest the next diagnostic tier.

Default budgets (can be overridden per fixture via calibrationBudgets in settings):

| Phase | Default budget |
| --- | --- |
| Warmup | 60 s |
| Discovery | 120 s |
| Mapping / Convergence | 180 s |
| Grid build | 10 s |
| Verification sweep | 30 s |
| Model fit | 15 s |
| Held-out gate | 30 s |

Total default budget: ~7.5 min, well above the typical 2–4 min runtime.

### B.5 Abort path

Cancellation can originate from three sources: operator (POST /api/calibration/mover/<fid>/cancel), phase timeout (§B.4), or an unhandled exception. All three converge on the same cleanup:

Foreground immediate blackout (operator-initiated only): on the /cancel request, the orchestrator zeroes the fixture's channel window on the running Art-Net engine buffer in the foreground, so the next 25 ms frame carries zeros to the bridge — the operator sees the light go off immediately.

Background unwind: the calibration thread sets _cancel_event, which _check_cancel() inside _hold_dmx picks up and raises CalibrationAborted. The exception propagates to _mover_cal_thread, which catches it, calls _cal_blackout() (512 zeros + release), sets status = "cancelled", phase = "cancelled".

Lock release — _set_calibrating(fid, False) is always called in the cleanup, regardless of which path triggered the cancel.

### B.6 Failure modes & operator diagnostics

| Symptom | Probable cause | What to check / try |
| --- | --- | --- |
| Discovery completes 80 probes without finding beam | Beam too dim, camera can't see it, wrong colour, mover not actually on | Verify fixture is powered and responding; increase threshold; run Fixture Orientation Test (§14); check §A.8 colour configuration; dim room lights |
| Discovery finds beam immediately but blink-confirm always rejects | Reflective surface (mirror, glass, polished floor) being detected instead of beam | Add diffuse material over the reflector; pick a different beam colour; move the mover's warmstart aim away from the reflector |
| Mapping / convergence aborts with "fewer than 6 samples" | BFS boundary is too narrow — camera sees only a small slice of the pan/tilt range | Reposition camera to see more of the floor; increase camera count; verify mover's position in layout matches reality |
| moverCalibrated flag never sets | Held-out gate is failing | Check the Accept/Retry prompt; if residual is reported, an Accept keeps the flag but marks it degraded; a full Retry restarts from discovery |
| Sign verification logs mirror ambiguity | Fit sees two equally good sign combos | Check physical mover pan/tilt direction against §14 Fixture Orientation Test; may need to toggle Invert Pan/Tilt or Swap Pan/Tilt flags |
| Calibration "hangs" at a phase | Phase budget (#653) not yet triggered; or Art-Net engine stopped mid-run | Wait up to the phase budget; check engine is running (POST /api/dmx/start if not); if still stuck, cancel and check orchestrator log |
| Light flashes momentarily then stops, status stays running | Foreground cancel happened but background thread is still unwinding | Normal; background cleanup completes within 1–2 s |

### B.7 Tuning-parameter reference

Operator-tunable (#680) — Settings → Advanced → Calibration Timeouts. The constants below are the shipped defaults; operators can override them via the Advanced panel, which persists into desktop/shared/data/settings.json under calibrationTuning. Overrides are read at phase start, so a change takes effect on the next calibration run without a server restart. Validation is centralised in CAL_TUNING_SPEC (parent_server.py); POST /api/settings rejects out-of-range values with a 400. Every constant below has a matching entry in CAL_TUNING_SPEC except the ones flagged "fixed" (e.g. STEP, MAX_SAMPLES is exposed as bfsMaxSamples, BRACKET_FLOOR is derived per fixture and not operator-tunable per #679).

Constants in desktop/shared/mover_calibrator.py:

| Constant | Default | Role |
| --- | --- | --- |
| SETTLE (legacy) | 0.6 s | Per-probe settle before detection |
| SETTLE_BASE (#655) | 0.4 s | Adaptive-settle base before escalation |
| SETTLE_ESCALATION | [0.4, 0.8, 1.5] s | Escalation tiers on pixel-drift retry |
| SETTLE_VERIFY_GAP | 0.2 s | Dual-capture spacing for median filter |
| SETTLE_PIXEL_THRESH | 30 px | Inter-capture drift threshold for "settled" |
| STEP | 0.05 | Fine spiral step size (normalized pan/tilt) |
| MAX_SAMPLES | 80 | Hard cap on BFS samples |
| COARSE_PAN | 10 | Legacy coarse grid pan bins |
| COARSE_TILT | 7 | Legacy coarse grid tilt bins |
| BRACKET_FLOOR | 1 / (2^pan_bits − 1) | Convergence refine floor — fixture-dependent (8-bit → ≈0.0039, 16-bit → ≈0.0000153) (#679) |

Constants in desktop/shared/mover_control.py:

| Constant | Default | Role |
| --- | --- | --- |
| Claim TTL (moverClaimTtlS, #680) | 15 s | Auto-release if claim not refreshed |

Phase time budgets (from CAL_TUNING_SPEC in desktop/shared/parent_server.py, #680):

| Key | Default | Clamp | Role |
| --- | --- | --- | --- |
| discoveryBattleshipS | 60 s | 20 – 300 | Coarse-grid scan budget |
| discoveryColourFallbackS | 90 s | 30 – 300 | Legacy colour-filter fallback budget |
| mappingS | 120 s | 30 – 600 | BFS mapping budget (soft cap) |
| fitS | 10 s | 5 – 60 | LM + grid build |
| verificationS | 30 s | 5 – 120 | Grid + parametric held-out verify |
| warmupSeconds | 30 s | 0 – 120 | Pre-cal motor warmup sweep |
| bfsMaxSamples | 80 | 20 – 300 | Hard cap on BFS samples |
| convergeMaxIterations | 25 | 5 – 100 | Per-target bracket-and-retry loop |
| battleshipPan/TiltStepsMin/Max | 3 / 8 / 3 / 6 | see spec | Adaptive-grid clamps (#661) |

### B.8 Related features

Unified mover control — see CLAUDE.md §"Unified mover control (gyro + Android)". Calibration must complete and moverCalibrated must be set before manual mover-control (gyro/phone) will use the parametric model; without it, the control layer falls back to raw DMX pan/tilt passthrough.

Fixture Orientation Test — §14 "Fixture Orientation Test". Run this first if the pan/tilt axes in the physical fixture don't match expectations.

Track Action — §8 "Track Action". Consumes the interpolation grid + parametric model to aim at moving subjects.

────────────────────────────────────────

## Appendix C — Documentation Maintenance

This appendix describes the contract between the calibration appendices above and the source code that implements them. It exists for issue #662 and is kept short — full details are in docs/DOCS_MAINTENANCE.md.

### C.1 Source-of-truth files

Any PR that changes calibration behaviour in one of these files is expected to include an Appendix A or B review in the same PR:

Mover calibration: desktop/shared/mover_calibrator.py, mover_control.py, parametric_mover.py, desktop/shared/spa/js/calibration.js, desktop/shared/parent_server.py (routes /api/calibration/mover/*).

Camera calibration: firmware/orangepi/camera_server.py, beam_detector.py, depth_estimator.py, desktop/shared/space_mapper.py, surface_analyzer.py, camera_math.py, desktop/shared/parent_server.py (routes /api/aruco/markers*, /api/cameras/<fid>/stage-map, /api/cameras/<fid>/aruco/*, /api/cameras/<fid>/intrinsic*, /api/cameras/<fid>/beam-detect, /api/space/scan).

### C.2 Reviewer checklist (short form)

On a calibration-touching PR, confirm:

Phase names in mover_calibrator.py match the Appendix B §B.2 table

Timeout constants in the §B.7 table still match code

Endpoint paths + request/response shapes in Appendix A match Flask route signatures

Rotation-schema v2 (§A.9) still matches camera_math.py::rotation_from_layout

Status strings written to the calibration-status dict match the state machine diagram

The full checklist, including render verification for the Mermaid diagrams under docs/diagrams/ and the DRAFT-banner removal criteria, is in docs/DOCS_MAINTENANCE.md.

### C.3 Regenerating the manual

Canonical source: docs/USER_MANUAL.md (this file).

docs/SlyLED_User_Manual.docx + .pdf are built separately by tests/build_manual.py, which constructs the document from scratch rather than parsing this markdown. The docx/PDF path does not yet include these appendices — follow-up work.

Diagram sources live in docs/diagrams/*.mmd. Mermaid blocks are embedded inline in the markdown so GitHub renders them directly; external renderers like Kroki can generate SVG/PNG from the standalone files for PDF inclusion.

### C.4 Enforcement

No automatic drift-check is wired up yet. Proposed options, in order of cost:

PR-template checkbox (.github/pull_request_template.md)

GitHub Actions grep: fail PRs that touch the source-of-truth list without touching docs/USER_MANUAL.md, with a skip-override label

Scheduled drift agent (weekly)

These require .github/ changes and are tracked as follow-ups under #662.

### C.5 DRAFT banner removal

The DRAFT banners on Appendix A and B should be removed once the in-flight items listed in docs/DOCS_MAINTENANCE.md §"When to bump the DRAFT banner" are all confirmed merged. At the time this appendix was drafted (2026-04-23), the following are known to be partial or not yet in code: #653 time budgets, #654 held-out parametric gate, #655 full median oversample, #658 blink-confirm on non-battleship path, #659 floor-view polygon target filter, #661 adaptive battleship density.

## Appendix D — Idle Behaviour

A "rock-solid" rig is one where, at any moment a show isn't playing and an operator isn't actively driving a head, every moving head parks at a known pose with the lamp closed. Operators expect that contract because the alternative — a head sitting on whatever pose the last writer happened to leave it in — looks broken even when nothing is wrong. This appendix is the canonical list of when the orchestrator parks, when it doesn't, and what "parked" means in practice.

### What "parked" means

A parked moving head:

Aims at its Home pose (the pose saved in the Set Home wizard;

fallback to mechanical centre when no Home is saved).

Holds dimmer = 0 so the lamp doesn't bleed onto the rig.

Closes the shutter if the profile carries a strobe channel

with a Closed capability — fixtures with mechanical shutters benefit from the explicit close.

Releases any colour-wheel slot it was holding back to slot 0

(open / white) so the next show frame doesn't inherit a stale filter.

### When the orchestrator parks a head

| Trigger | Path | Notes |
| --- | --- | --- |
| Cold start | Orchestrator boot | Every DMX fixture parks once the engine comes up. Avoids the "head was left aimed at the back wall last night" surprise. |
| Timeline natural end | _dmx_playback_loop exit | Heads driven by the timeline's bake park when the show finishes. Track-action-driven heads also park (#807) — pre-fix only the bake-driven heads parked, leaving any tracker-claimed mover stuck at its last pose. |
| Operator presses Stop | _dmx_playback_stop set | Same as natural end. The blackout sweep applies only on stop or final-iteration end (#840), not between loop iterations. |
| Claim release | Mover-control claim arbiter | When an Android phone or gyro controller releases a claim, the head returns to the show if a show is running, otherwise parks. The release is instant — no slewed easing in v1.7.83+. |
| Power-cycle re-settle | First PONG from a child after a boot | When a fixture's child board power-cycles, the orchestrator resends the current globalBrightness (#843) and the next show frame writes a known pose. Pre-v1.7.83 the child could come up at full brightness for one frame; the PONG-time top-up closes that window. |

### What does NOT trigger a park

These intentionally do not park heads — operators sometimes expect them to, but parking on every one-shot would steal the head from the show.

One-shot `/api/mover/<fid>/aim` — these are operator-direct

test pulses; the rig holds whatever pose the route sets until the next show frame.

DMX-test sliders on the Settings tab — same reasoning. The

sliders override show output for as long as the operator is driving them.

Brief gaps within an active claim — a phone or gyro temporarily

losing WiFi for a second doesn't release the claim. The remote- control claim TTL is 15 s; a head only parks when the TTL elapses without a heartbeat (#813 §6.3 "all-comms silence").

Calibration probes — beam discovery and convergence sweeps

hold the head wherever the probe lands. The calibration session parks the head explicitly when it finishes (success or abort).

### How to verify on your rig

Park a moving head at a known wall (Set Home → save).

Stop any running show.

Aim the head at the floor with /api/mover/<fid>/aim.

Press play on a 5-second blank timeline. The head should NOT

move (rule three: one-shot aims hold).

Stop the timeline. The head should return to Home with the lamp

closed within ~50 ms.

If step 5 doesn't happen, check (a) the fixture has a Home pose saved, (b) the timeline ended naturally rather than crashing, (c) no claim is held on the fixture (the Setup tab shows claim state per fixture).

────────────────────────────────────────

## Appendix E — Remote Control: Android Phone & Gyro Gyro

Two remote controllers can drive moving heads in real time alongside a running show: an Android phone running the SlyLED operator app and a Waveshare ESP32-S3 round-LCD gyro controller. Both go through the same claim arbiter on the orchestrator, both follow the same handshake protocol, and both cooperate with the show timeline through the mover-control claim arbiter. This appendix describes the full lifecycle, the gestures, and how claim arbitration interacts with preset shows.

### Claim lifecycle

The claim is the orchestrator's "this remote currently owns mover N" lock. It carries a 16-bit nonce, a TTL, and a current pose so the system can reconcile the gyro's UI state with the orchestrator's arbiter state when one of them reboots or drops a packet.

1. IDLE on remote.
2. Operator presses Start (gyro) or Claim (Android).
3. Remote ships CMD_GYRO_START / claim request with a fresh 16-bit nonce.
4. Orchestrator allocates a mover, replies CLAIM_ACK with the
   nonce + assigned moverId. The remote advances UI to ACTIVE
   only on a matching ACK; CLAIM_DENIED reverts; ~1.5 s overall
   timeout reverts with "NO RESPONSE".
5. Remote streams orient quaternions at ~50 Hz; orchestrator
   converts to aim-stage and writes pan / tilt to the head.
6. Both ends exchange 2 s heartbeats (HB_REP carries uiState +
   claimNonce + seq) so divergent state is reconciled.
7. Operator presses Stop / Release; remote ships nonce; orchestrator
   replies STOP_ACK and releases the claim.

The full state-machine spec lives in docs/gyro-claim-lifecycle.md and is the source of truth for any change to the protocol.

#### What the operator sees

Press Start on the gyro — page advances to "ACTIVE" within

~150 ms. If the orchestrator can't claim a mover (none online, none available), the page reverts to IDLE with a denial reason.

Press Stop on the gyro — page returns to IDLE; the head

returns to whatever the show was driving (or parks if no show is running).

Calibrate — hold the Calibrate button (gyro or Android)

for as long as you need; release to capture the new reference pose. The screen advances to the colour picker page on the gyro; the Android app advances to the gesture page.

Connection lost — both remotes show a stale-reason badge if

the orchestrator stops hearing heartbeats. The gyro self-clears when it resumes streaming (#812 / #821 / #823); operator can also force-clear via POST /api/remotes/<id>/clear-stale.

### Gestures

Once active, both remotes drive the same aim_stage semantic — the head's beam aims at a stage-coordinate point computed from the remote's orientation.

#### Phone (Android)

Pitch (tip phone forward / back) — beam pitches up / down on

the head.

Roll (tilt phone left / right) — beam pans across the stage.

Yaw (rotate phone around vertical) — beam pans across the

stage.

Volume buttons — fine dimmer up / down (Android operator app

configurable).

Auto Brightness (chapter on Brightness) — the app can drive

the orchestrator's master brightness from the local mic envelope at ~20 Hz, gamma-scaled to the rig (#820, #843).

The phone-specific yaw axis is mirrored relative to the gyro (#824) because the phone's natural-portrait orientation puts the operator's "left" 90 ° offset from the gyro's body frame. The operator never needs to think about this; the orchestrator's _apply_quat for KIND_PHONE handles the negation.

#### Gyro gyro

Pitch (tip gyro forward / back) — beam pitches up / down.

Yaw (rotate around the gyro's vertical axis) — beam pans.

Roll (tilt left / right) — colour-wheel selection on profiles

with a colour wheel; ignored on RGB-only profiles.

Press Start — claim mover and start streaming.

Press Stop — release claim.

Press Calibrate — capture new reference pose.

### Claim arbitration with shows (#763)

Claims take priority over the show timeline:

A claimed head is muted from _evaluate_track_actions and

from the bake-driven set_fixture_dimmer / set_fixture_pan_tilt writes. The claim writer owns the head until release.

Other heads in the rig keep playing the show normally — the

claim affects only the assigned mover.

On release the head rejoins the show within one frame: no

slew, no fade. If the show has moved on, the head jumps to wherever the show currently has it. This was a deliberate choice per #763 — slewing back in took the operator out of the moment; snap-rejoin is what real consoles do.

Track actions evaluate every frame, so a head that was claimed

during a sweep returns to wherever the sweep is right now, not to where the sweep would have been mid-claim.

### Colour & dimmer during a claim (#814)

A claim doesn't take over colour or dimmer:

The remote's gestures drive only pan / tilt (and colour wheel

for the gyro's roll axis on profiles that support it).

The head's dimmer and RGB stay under the show's control. If the

show is dim, the claimed head stays dim — the operator picks pan and tilt; the show paints colour and intensity.

This applies to global brightness (#843) the same way: a claim

during Auto Brightness inherits the auto-driven master.

### Recovery from divergent state

The handshake's heartbeats include both ends' state, so divergent combinations are reconciled:

| Gyro UI | Orchestrator | What happens |
| --- | --- | --- |
| ACTIVE | claim held | Normal — heartbeats keep TTL alive. |
| ACTIVE | no claim | Orchestrator reconstructs the claim (orchestrator-restart bootstrap). |
| IDLE | claim held | Orphan claim — orchestrator releases it. |
| IDLE | no claim | Normal idle. |

The orphan-claim guard fires 1.5 s after CLAIM_ACK if no orient arrives, releasing the claim so a wedged remote can't squat on a mover indefinitely.

────────────────────────────────────────