# SlyLED Manuel d'utilisation

Système d'éclairage volumétrique 3D — v1.0.0

Guide complet pour concevoir, programmer et exécuter des spectacles lumineux LED + DMX avec l'orchestrateur SlyLED, les performers et le pont DMX.

electricrv.ca/slyled

# Table des matières

1. Getting Started with 3D Stage Design

2. Fixture Setup

3. Creating Spatial Effects

4. Building a Timeline

5. Baking & Playback

6. Show Preview Emulator

7. Preset Shows

8. DMX Fixture Profiles

9. Settings & Configuration

10. Camera Setup & Calibration

11. 3D Environment Scanning

12. Stereo 3D & Light Mapping

13. Project Files

14. System Limits

15. Troubleshooting

16. Examples

17. API Quick Reference

# 1. Démarrer avec la conception de scène 3D

## Aperçu

SlyLED is a three-tier lighting system: the Orchestrator (Windows/Mac desktop app) designs and controls shows, Performers (ESP32/D1 Mini boards with LED strings) execute lighting effects, and DMX Bridges (Giga R1 boards) drive professional DMX fixtures over Art-Net.

Dashboard — live performer status overview

## Basculer entre 2D et 3D

The Layout tab offers two views:
- 2D Canvas: Flat top-down layout for simple setups
- 3D Viewport: Interactive Three.js scene for complex multi-level installations

Both views share the same position data. Switching is instant and non-destructive.

Layout tab — 2D Canvas with placed fixtures

Layout tab — 3D Viewport with stage wireframe, fixtures, and objects

## Naviguer dans la vue 3D

| Action | Control |
| --- | --- |
| Orbit (rotate) | Left-click + drag |
| Zoom | Scroll wheel |
| Pan (shift view) | Right-click + drag |
| Select fixture | Left-click on node |
| Move fixture | Drag the colored arrows |
| Edit fixture | Double-click on node |
| Place from sidebar | Drag unplaced fixture into viewport |

## Système de coordonnées

- X-axis (red): Width — left to right
- Y-axis (green): Height — ground to ceiling
- Z-axis (blue): Depth — front to back
- Origin: Bottom-left-front corner of the stage
- Units: Internally millimeters; displayed in Settings-chosen unit

# 2. Configuration des projecteurs

## Que sont les projecteurs ?

A fixture is the primary entity on the 3D stage. It wraps physical hardware and adds stage-level attributes like position, rotation, and aim point.

- LED Performers: Auto-created when hardware is registered via Setup tab
- DMX Fixtures: Created manually with universe, address, and profile assignment
- Fixtures can override child attributes (e.g., rotate a horizontal string to vertical)
- The baking engine uses the fixture's position and rotation, not the child's raw config

Setup tab — LED and DMX fixtures with type badges and status

## Ajouter un performer LED

Click Add Fixture, select "SlyLED Performer (LED)", and enter the device IP address. The system probes the device via UDP PING and HTTP, registers it as a child, and auto-creates a linked fixture.

Add Fixture — LED performer flow with IP address entry

## Ajouter un projecteur DMX

Click Add Fixture, select "DMX Fixture", and configure:
- Name: Descriptive label
- Universe: DMX universe number (1+)
- Start Address: DMX start channel (1-512)
- Channel Count: Number of channels the fixture uses
- Profile: Select from the built-in library or import from Open Fixture Library

DMX fixtures appear in the setup table with purple "DMX" badges.

Add Fixture — DMX fixture with profile dropdown

## Modifier les projecteurs DMX

Click Edit on a DMX fixture to modify its properties. The edit modal shows:
- Universe, Start Address, Channel Count
- Profile ID (for channel name/type mapping)
- Aim Point (X, Y, Z in mm) — where the beam points for moving heads
- Test Channels — interactive sliders to control each DMX channel live
- Rotation Override (degrees)

Edit DMX fixture — aim point, profile, and test channels

## Types de projecteurs

| Type | Description | Use Case |
| --- | --- | --- |
| Linear | LED strip/string with pixels along a path | LED performers |
| Point | Single light source with area of effect | DMX pars, spots, moving heads |
| Object | 3D mesh as a projection target | Walls, screens |
| Group | Named collection of fixtures | Zones, grouped control |

# 3. Créer des effets spatiaux

Spatial effects operate in 3D space. A sphere of colored light sweeping across the stage illuminates different fixtures at different times based on their physical position. For DMX moving heads, the effect center becomes the aim target — heads track the effect as it moves.

Actions tab — spatial effects and classic action library

## Champs spatiaux

| Field | Description |
| --- | --- |
| Shape | Sphere, Plane, or Box |
| Color | RGB color applied to pixels inside the field |
| Size | Radius (sphere), thickness (plane), or W/H/D (box) |
| Motion Start | Starting position [x, y, z] in mm |
| Motion End | Ending position [x, y, z] in mm |
| Duration | Travel time from start to end |
| Easing | Linear, ease-in, ease-out, ease-in-out |
| Blend | Replace, Add, Multiply, or Screen |

## Lyres + effets spatiaux

When a spatial effect is applied to a DMX moving head fixture:
- The effect's center position becomes the aim target
- Pan/tilt angles are computed from fixture position to aim point
- Color is applied from the effect's RGB values
- Moving effects produce time-sliced segments that track the motion
- 3D viewport shows beam cones pointing at the effect center

## Types d'actions DMX

In addition to classic LED actions (Solid, Chase, Rainbow, etc.), four DMX-specific action types are available for direct control of DMX features:

| Action Type | Description |
| --- | --- |
| DMX Scene | Set exact values for dimmer, pan, tilt, strobe, gobo, color wheel, prism |
| Pan/Tilt Move | Animate pan/tilt from start to end position over time |
| Gobo Select | Select a gobo wheel position with optional color |
| Color Wheel | Select a color wheel position |

Classic LED actions (Solid, Breathe, Chase, etc.) are automatically converted to DMX Scene segments when assigned to DMX fixtures. The dimmer is auto-set to 255 when color is active, and pan/tilt default to center (0.5).

# 4. Construire une timeline

Timelines are multi-track, overlapping effect sequences with precise timing. Use the Runtime tab to create and edit timelines.

Runtime tab — timeline editor

## Créer une timeline

1. Click "+ New Timeline" and enter name and duration
2. Select the timeline from the dropdown
3. Add tracks (one per fixture or "All Performers")
4. Add clips referencing spatial effects or classic actions
5. Adjust clip timing by editing start time and duration

# 5. Compilation et lecture

## Qu'est-ce que la compilation ?

Baking compiles a timeline into minimal action instructions for each performer. The smart bake engine analyzes each clip's spatial geometry directly and computes per-string sweep patterns, directions, and speeds.

For DMX fixtures, baking also computes:
- Pan/tilt angles from spatial effect motion paths
- Color values from the effect's intersection with the fixture position
- Dimmer values (auto 255 when color is active)
- Time-sliced segments for smooth moving head tracking

## Lecture

After baking and syncing:
- LED performers receive action steps via UDP and execute locally
- DMX fixtures are driven by a 40Hz playback loop sending Art-Net/sACN packets
- All channels (RGB, pan, tilt, dimmer, strobe, gobo) are sent per profile
- 16-bit channels (pan/tilt) are split into coarse + fine bytes automatically

# 6. Émulateur d'aperçu du spectacle

The emulator shows a real-time preview of your show on the Runtime tab.

- LED fixtures: Colored dots along string directions, animated per-pixel
- DMX fixtures: Beam triangles from fixture position toward aim point
- Beam color and alpha reflect the fixture's current DMX state
- Time counter shows elapsed position

# 7. Spectacles prédéfinis

14 themed shows are available from the Runtime tab. Shows are dynamically generated based on your actual fixtures — every fixture gets coverage with no dark periods. LED fixtures get pattern effects, DMX pars get color washes, and moving heads get pan/tilt sweeps that track spatial effects across the stage.

Each load produces a unique variation of the theme with randomized timing and positions.

| Preset | Type | Description |
| --- | --- | --- |
| Rainbow Up | Spatial plane | Rainbow from floor to ceiling |
| Rainbow Across | Spatial sphere | Rainbow sweeping left to right |
| Slow Fire | Classic action | Warm fire effect on all fixtures |
| Disco | Classic action | Pastel twinkle sparkles |
| Ocean Wave | Spatial (2 effects) | Blue wave sweep with teal wash |
| Sunset Glow | Mixed | Warm breathe with golden plane sweep |
| Police Lights | Mixed | Red strobe with blue box flash sweep |
| Starfield | Classic action | White sparkles on dark background |
| Aurora Borealis | Spatial (2 effects) | Green curtain with purple shimmer |
| Spotlight Sweep | Spatial (moving heads) | Warm orb sweeps stage — heads track it |
| Concert Wash | Mixed (moving heads) | Magenta flood + amber tracking spot |
| Figure Eight | Spatial (moving heads) | Crossing orbs — heads trace X paths |
| Thunderstorm | Mixed (moving heads) | Lightning strikes — heads chase bolts |
| Dance Floor | Mixed (moving heads) | Fast orbiting spots — rapid tracking |

# 8. Profils de projecteurs DMX

Profiles define the channel layout and capabilities of DMX fixtures. Each channel maps a DMX offset to a function (pan, tilt, red, green, blue, etc.) with capability ranges that describe what each DMX value range does.

Fixture Profile Library — 12 built-in profiles

## Voir un profil

Click View on any profile to see the full channel table with capabilities. Moving head profiles show pan/tilt ranges, beam width, and color mode.

Moving Head 16-bit profile with 13 channels and capability ranges

## Créer des profils personnalisés

Click "New Profile" to open the editor. Define channels with type, name, bits (8/16), and capability ranges. Click the capabilities button on each channel to define what DMX value ranges do (e.g., gobo positions, strobe speeds, color wheel slots).

Profile editor — channel table with type dropdowns

Capability range editor for a channel

## Importer depuis l'Open Fixture Library

Click "Import OFL" to paste fixture JSON from open-fixture-library.org. The importer converts OFL's capability model to SlyLED format, handling multi-mode fixtures, 16-bit channels, and color detection automatically.

Open Fixture Library import modal

## Import/export de lots

Export your custom profiles as a JSON bundle file for backup or sharing. Import bundles to add profiles from other installations.

# 9. Paramètres et configuration

Settings tab — app configuration and DMX output

The Settings tab contains:
- Orchestrator name and units
- Stage dimensions
- Dark mode toggle
- Logging control
- Configuration export/import
- Show export/import
- DMX output settings (Art-Net/sACN, frame rate, universe routing)
- Fixture Profile Library
- Factory reset

## Firmware

Firmware tab — board detection and flash controls

The Firmware tab provides:
- COM port detection with board type identification
- Serial version query
- WiFi credential management
- One-click firmware flashing for ESP32, D1 Mini, and Giga R1 boards

# 10. Configuration des caméras et calibration

## Ajouter des nœuds caméra

Camera nodes are Orange Pi or Raspberry Pi SBCs running the SlyLED camera firmware. They capture snapshots for calibration, beam detection, depth estimation, and tracking.

To add a camera:
1. Connect the camera node to the same network as the orchestrator
2. Go to Setup tab and click "Discover" — camera nodes respond to UDP broadcast
3. Or manually add via Settings > Cameras with the camera's IP address
4. Each USB camera sensor appears as a separate fixture that can be placed in the Layout

Settings > Cameras — camera list with calibration status

## Calibration par marqueurs ArUco

ArUco markers are used for camera intrinsic calibration (lens parameters).

Step-by-step:
1. Print ArUco markers: Settings > Cameras > "Print ArUco Markers" (6 markers, 150mm each)
2. Place printed markers on flat surfaces visible to the camera at various angles
3. Click "Capture" 5+ times from different positions — each capture detects markers automatically
4. Click "Compute" — the system runs cv2.calibrateCamera() to determine fx, fy, cx, cy
5. RMS error < 2.0 indicates good calibration

All ArUco detection runs on the orchestrator PC (not the camera hardware), so any camera that can serve JPEG snapshots works — even low-end Raspberry Pi boards.

## Carte de scène — Pose de la caméra

The Stage Map determines each camera's 3D position and orientation in stage coordinates using solvePnP with ArUco markers at known stage positions.

Step-by-step:
1. Place 3+ ArUco markers on the stage floor at known positions (measure with tape)
2. Enter each marker's stage coordinates (X, Y, Z in mm) in the wizard
3. Click "Compute Stage Map" — the system detects markers and solves for camera pose
4. Result: camera position in stage mm, floor-plane homography, reprojection error

The homography maps any pixel to stage floor coordinates (X, Y at Z=0), enabling beam detection in real-world millimeters rather than pixel space.

## Assistant de calibration des lyres

Moving head fixtures need calibration to map DMX pan/tilt values to real-world positions.

The wizard runs automatically:
1. Discovery: Spiral search to find the beam in the camera's field of view
2. Adaptive settle: Waits 0.8-2.5s per move, verifies beam has stopped (double-capture)
3. BFS mapping: Explores visible region, stops at boundaries when beam leaves camera view
4. Grid build: Creates a bilinear interpolation grid for fast pan/tilt lookup
5. Boundary verification: Flash on/off at edges to confirm true visibility limits

After calibration, the system can aim any fixture at any stage coordinate using grid_inverse().

# 11. Scan 3D de l’environnement

## Capture du nuage de points

The environment scan creates a 3D point cloud of your venue using depth estimation.

Step-by-step:
1. Position and calibrate 2+ cameras (Stage Map required)
2. Go to Settings > Space > "Start Scan"
3. Each camera captures a snapshot, the CV Engine runs Depth-Anything-V2 locally
4. Point clouds from each camera are transformed to stage coordinates and merged
5. Floor is auto-detected and normalized to Z=0

Result: 5,000-20,000 colored 3D points representing your venue geometry.

## Détection des surfaces

After scanning, click "Analyze Surfaces" to run RANSAC-based detection:

- Floor: Horizontal plane detection (normal near vertical)
- Walls: Vertical planes along stage boundaries
- Obstacles: Clustered points that don't belong to floor or walls (pillars, furniture)

Detected surfaces are used by the calibration system to predict where beams land and by the structured-light refinement to verify 3D accuracy.

## Moteur de vision par ordinateur

All computer vision processing runs on the orchestrator PC, not the camera hardware. The CV Engine wraps three model pipelines:

- Beam Detection: Fast color-filtered bright spot detection (<100ms)
- Depth Estimation: Depth-Anything-V2 monocular depth (1-2s on x86)
- Object Detection: YOLOv8n for person/object detection (100-200ms)

Camera nodes only provide JPEG snapshots. This means any camera works — even a Raspberry Pi 3 that can't run ONNX models locally.

CV Engine status — model loading state in Settings

# 12. Stéréo 3D et cartographie lumineuse

## Triangulation stéréo

With 2+ calibrated cameras, the stereo engine triangulates 3D points from pixel observations. Each camera converts a pixel to a 3D ray; the intersection of rays from different cameras gives the 3D position in stage mm.

Use cases:
- Verify fixture positions by observing them from multiple cameras
- Track moving objects (people) in 3D
- Cross-validate depth estimation against triangulated ground truth

Accuracy depends on camera baseline (distance between cameras) and calibration quality. Typical error: 20-100mm at 3-5m distance with 60° FOV cameras.

## Cartographie lumineuse par projecteur

A light map records where each moving head's beam lands at every pan/tilt position in real stage coordinates.

Step-by-step:
1. Calibrate the fixture (mover calibration wizard)
2. Click "Build Light Map" on the fixture
3. The system sweeps the beam across the visible area in a grid pattern
4. At each position: detect beam pixel, convert to stage mm via homography
5. Result: (pan, tilt) → (stageX, stageY, stageZ) lookup table

Inverse lookup: given a target position on stage, find the exact pan/tilt to aim there. Uses inverse-distance weighted interpolation of the 4 nearest samples.

## Faisceau comme lumière structurée

During calibration sweeps, each beam position that hits a surface provides a definite 3D contact point. These points refine the environment model:

- Floor height correction: Beam contacts adjust the detected floor Z value
- Wall boundary extension: Contacts outside known wall boundaries expand the model
- Surface verification: Predicted ray-surface intersections are compared with observed positions

This is more accurate than depth estimation alone because the beam position is precise (bright centroid) and the ray direction is known (from pan/tilt geometry).

# 13. Fichiers de projet

## Exportation

File > Export saves your entire project as a .slyshow file containing:

- All fixtures, children, layout positions, and stage dimensions
- Actions, spatial effects, timelines, and show playlist
- DMX settings, fixture profiles, and calibration data
- Camera calibrations (intrinsics, stage maps, mover grids)
- Point cloud (gzip-compressed for portability)
- Light maps (per-fixture pan/tilt-to-stage lookup tables)

The project file uses schema version 2. Files from older versions (v1) import successfully — new spatial fields default to empty.

## Importation

File > Import loads a .slyshow file, replacing all current state:

1. All playback is stopped
2. Fixtures, actions, timelines, and objects are restored
3. Calibration data (homographies, grids, light maps) is restored
4. Point cloud is decompressed and loaded into the 3D viewport
5. Camera SSH passwords must be re-entered (not portable between machines)

This enables a workflow where a designer builds the show on their laptop, exports the project, and a runtime operator loads it on the show PC — all spatial data transfers automatically.

## Données spatiales dans les projets

Point clouds can be 10,000-20,000 points (1-3 MB uncompressed). The export pipeline gzip-compresses the point array to ~200-500 KB. On import, the compressed data is automatically decompressed and loaded.

Light maps (typically 200-300 samples per fixture) are stored alongside mover calibration data.

# 14. Limites du système

| Resource | Limit | Notes |
| --- | --- | --- |
| Children (performers) | 8 max | Protocol constant |
| Strings per child | 8 max | ESP32 supports up to 8 GPIO pins |
| LEDs per string | 65535 max | uint16_t addressing (protocol v4) |
| Steps per runner | 16 max | LoadStepPayload array limit |
| Bake segments per fixture | 64 max | Supports PT move time slices |
| Bake frame rate | 40 Hz | Art-Net output rate |
| DMX universes | Unlimited | One Art-Net packet per universe |
| DMX channels per universe | 512 | DMX-512 standard |
| Pan/tilt resolution | 8 or 16 bit | Per profile channel definition |
| Preview resolution | 1 fps | 1 color per string per second |
| NTP sync offset | 5 seconds | GO command sent with future epoch |

# 15. Dépannage

## 3D Viewport Not Rendering

Browser doesn't support WebGL. Use Chrome, Firefox, or Edge.

## Performers Not Syncing

Children offline or on different network. Check Setup tab status.

## Bake Error: "No fixtures"

Add and position fixtures in Layout before baking.

## DMX Not Outputting

Check Art-Net engine is started (Settings → DMX Output → Start). Verify universe routing matches your bridge address.

## Moving Heads Not Tracking

Verify fixture has a profile with panRange/tiltRange > 0. Check rotation is set (Edit fixture → Pan/Tilt fields).

## Preview Shows No DMX Beams

Bake the timeline first. DMX preview requires baked data.

## Factory Reset

Settings → Factory Reset clears ALL data including fixtures, effects, timelines, and profiles.

# 16. Exemples

These worked examples walk through common workflows from start to finish. Each example includes the exact steps, configuration values, and screenshots showing what you should see at each stage.

## Exemple A : Suivi caméra — Les lyres suivent une personne

Make DMX moving heads automatically follow people detected by a camera.

### Prérequis

- At least one camera node online (Firmware tab -> deploy + verify)
- At least one DMX moving head fixture placed on the Layout tab
- Moving head profile configured with pan/tilt range
- Art-Net/sACN engine running (Settings -> DMX -> Start)
- Mover calibration completed (see Example C) for accurate aiming

### Étapes

1. Verify hardware - Open the Setup tab. Confirm movers show green status and camera nodes show online. If cameras are offline, check WiFi and deploy firmware from the Firmware tab.

2. Start camera tracking - Go to the Layout tab. Click a camera fixture to select it. Click the Track toggle button. The status bar shows "Tracking active." The camera node begins running YOLO person detection at 2 fps and creates temporal stage objects for each detected person.

3. Create a Track action - Go to the Actions tab. Click + New Action:
   - Name: Person Follow
   - Type: Track (last option in the dropdown)
   - Color: Pick the beam color (e.g. red for a spotlight)
   - Leave Target Objects empty (follows ALL detected people)
   - Cycle Time: 2000 ms
   - Check Fixed assignment for strict 1:1 mapping

4. Create a timeline - Go to the Shows tab. Click + New Timeline, name it "Person Tracking", set duration to 600s, enable Loop.

5. Start playback - Click Bake, then Start. The 40 Hz DMX playback loop begins.

6. Test - Walk in front of the camera. Within 2 seconds a pink person marker appears in the 3D viewport. The moving heads aim at you.

### Comportement d’attribution

| Personnes visibles | Avec 2 lyres |
| --- | --- |
| 1 person | Both heads aim at the same person |
| 2 people | One head per person (1:1) |
| 3+ people (cycling) | Heads cycle through people every 2s |
| 3+ people (fixed) | First 2 tracked, 3rd ignored |

## Exemple B : Suivi de lyres avec effets spatiaux

Make moving heads follow a virtual object sweeping across the stage. No camera or physical hardware required — this runs entirely in the emulator.

### Partie 1 — Scène et configuration des projecteurs

1. Set stage dimensions - Open the Settings tab. Under Stage, enter:
   - Width: 6000 mm, Height: 3000 mm, Depth: 4000 mm
   - Click Save

2. Create a DMX profile - Go to Settings -> Profiles -> New Profile:
   - Name: Narrow Spot
   - Beam Width: 8 degrees
   - Pan Range: 540, Tilt Range: 270
   - Channels: Pan (16-bit), Tilt (16-bit), Dimmer, Red, Green, Blue
   - Click Save Profile

Éditeur de profil — Faisceau étroit 8 degrés

3. Add two movers - Setup tab -> + Add Fixture (twice):
   - Mover SL: Universe 1, Address 1, profile Narrow Spot
   - Mover SR: Universe 1, Address 14, profile Narrow Spot

### Partie 2 — Disposition 3D et effet spatial

4. Position movers on the truss - Layout tab -> drag each mover:
   - Mover SL: X=1500, Y=0, Z=2800 (stage left on truss)
   - Mover SR: X=4500, Y=0, Z=2800 (stage right on truss)
   - Switch to 3D view to confirm both are elevated and aimed downward.

Onglet Disposition — Vue 3D avec deux lyres sur la poutre

5. Create a spatial effect - Actions tab -> + New Action:
   - Name: Sweep Green
   - Type: Spatial Effect, Shape: Sphere, Radius: 800 mm
   - Color: Green (0, 255, 0)
   - Motion Start: X=1000, Y=2000, Z=0
   - Motion End: X=5000, Y=2000, Z=0
   - Duration: 8 seconds, Easing: Linear
   - This creates a green sphere that sweeps left to right in 8 seconds.

Onglet Actions — Effet spatial Balayage Vert

### Partie 3 — Timeline, compilation et lecture

6. Create a timeline - Shows tab -> + New Timeline:
   - Name: Mover Tracking Demo, Duration: 20s, Loop: On
   - Add track targeting All Performers
   - Add clip referencing Sweep Green, 0s start, 8s duration

Onglet Spectacles — Timeline avec clip d’effet spatial

7. Bake the timeline - Click Bake. The engine computes per-fixture pan/tilt angles for each 25ms frame.

8. Start playback - Switch to Runtime tab. Click Start. Both beam cones animate in real-time, tracking the green sphere across the stage.

Lecture — Cônes de faisceau au départ (T=0s)

Lecture — Faisceaux en suivi à mi-parcours (T=5s)

Lecture — Faisceaux en position finale (T=10s)

What to look for:
- Both beam cones should be green (matching the effect color)
- The cones should move smoothly from left to right
- Beam opacity > 0 during the sweep, indicating active output
- If beam cones don't appear, ensure fixtures are positioned and the timeline is baked

## Exemple C : Calibration des lyres

Calibrate a moving head so the system knows exactly where its beam lands for any pan/tilt position. This two-part process discovers the visible range then builds a light map for inverse lookup.

### Prérequis

- Camera node positioned on Layout with camera calibration complete (Example D)
- Moving head fixture placed on the Layout tab
- Art-Net engine running
- Dim ambient lighting (beam must be visible to camera)
- Beam aimed at the floor within the camera's field of view

### Partie 1 — Découverte pan/tilt et calibration de grille

1. Open calibration - Layout tab -> double-click a moving head -> click Calibrate.

Panneau de calibration — Nom du projecteur et options

2. Choose beam color - Select a color that contrasts with your floor:
   - Green on dark floors, Magenta on light floors

3. Run discovery - Click Start Calibration. The system sweeps a coarse 8x5 grid across the full pan/tilt range. The camera watches for the beam on the floor.
   - Phase 1: Coarse grid scan (~40 positions)
   - Phase 2: Fine spiral refinement from detected position

Découverte en cours — Scan de grille grossière

4. BFS mapping - The system explores the visible region in 4 directions from the discovered position. Collects up to 60 samples using adaptive settle times (0.8-2.5s per move). Mapping stops at boundaries.

5. Grid build and review - The calibration summary shows:
   - Sample count (aim for 30+)
   - Pan/tilt range (normalized 0.0-1.0)
   - Grid density

Calibration de grille terminée — Nombre d’échantillons et plage

### Partie 2 — Carte lumineuse (scène vers pan/tilt)

6. Build light map - Click Build Light Map. The system sweeps a 20x15 grid:
   - At each position: move, detect beam, convert pixel to stage mm via homography
   - Builds a (pan, tilt) -> (stageX, stageY, stageZ) lookup table
   - Typical time: 5-10 minutes for 300 samples

Construction de la carte lumineuse — Balayage en cours

7. Verify inverse lookup - Click Aim and enter a target stage position:
   - The system uses inverse-distance interpolation to find the pan/tilt
   - Verify the beam lands within 100-200mm of the target
   - Test 3-4 positions across the stage

8. Save - Calibration data is auto-saved with the fixture and included in project exports.

Vérification de visée — Faisceau sur la position cible

### Calibration manuelle (sans caméra)

If automated calibration isn't available:

1. Double-click mover -> Manual Calibrate
2. Define 4-6 physical markers at known stage positions
3. Jog beam to each marker using pan/tilt sliders, click Record
4. Click Compute — the system fits a 3D affine transform
5. The transform extrapolates beyond calibrated points

Re-calibrate when: fixture moved, venue changed, pan/tilt range updated, or accuracy degrades.

## Exemple D : Calibration caméra avec marqueurs ArUco

Calibrate a camera so pixel coordinates map to real stage positions. This is a prerequisite for beam detection, person tracking, and mover calibration.

### Prérequis

- Camera node online and reachable on the network
- Camera fixture registered (Setup -> Discover, or Settings -> Cameras)
- Camera placed on the Layout tab at its physical position
- Printer for ArUco marker sheet (A4/Letter)
- Tape measure for recording marker positions on stage

### Partie 1 — Préparer et placer les marqueurs

1. Print ArUco markers - Settings -> Cameras -> Print ArUco Markers. A modal shows 6 ArUco 4x4 markers (IDs 0-5), each 150mm.
   - Print at 100% scale (no fit-to-page)
   - Card stock is more durable than regular paper

Impression des marqueurs ArUco — 6 marqueurs

2. Place markers on the stage floor:
   - Minimum 3 markers, recommended 4-6
   - Spread across the camera's field of view
   - Place flat on the floor (tilted markers reduce accuracy)
   - Measure each marker's X, Y position from the stage origin (mm)

### Partie 2 — Enregistrer et positionner la caméra

3. Register camera - Setup tab -> Discover, or Settings -> Cameras -> add IP.
   Each USB camera sensor appears as a separate fixture.

Configuration caméra — Liste avec IP et statut

4. Position in 3D - Layout tab -> drag camera to its physical position.
   Set rotation to match aim direction. The camera frustum should cover the markers on stage.

### Partie 3 — Calibrer et vérifier

5. Run calibration - Layout tab -> click camera -> Calibrate:
   - Camera captures a frame, ArUco markers are auto-detected
   - For each marker: enter the real-world X, Y coordinates, click Record
   - Click Compute — builds a homography matrix (pixels -> stage coords)

Marqueurs ArUco détectés — Cadres verts avec identifiants

6. Review results:
   - Reprojection error: < 10mm excellent, 10-20mm good, > 50mm re-try
   - Reference points: should match markers recorded
   - Coverage area: larger is better

7. Save - Click Save. The camera badge shows a green checkmark.
   All tracking and detection features now use stage coordinates.

Calibration terminée — Erreur et couverture

### Conseils

- Use large markers (> 10 cm) for reliable detection at distance
- Place markers at the extremes of the camera's view, not just center
- Avoid direct glare on glossy printed markers
- Re-calibrate when: camera moved, lens changed, or stage reconfigured

## Exemple E : Spotlight suivi de personne — Spectacle de suivi en direct

Use the built-in Spotlight: Follow Person preset to make moving heads automatically follow people detected by a camera in real-time.

### Prérequis

- Camera node online with person detection working
- DMX moving head fixtures placed on the Layout tab
- Camera calibration complete (Example D)
- Mover calibration complete (Example C)
- Art-Net engine running

### Étapes

1. Load the preset - Runtime tab -> Load Show -> Spotlight: Follow Person.
   Warnings appear if camera or movers are missing (preset still loads).

2. What it creates:
   - Track action (type 18) on all movers targeting "person" objects
   - Warm spotlight color (255, 240, 200) at full dimmer
   - Dim blue ambient wash on LED fixtures for framing
   - 10-minute looping timeline for the DMX playback loop

3. Start camera tracking - Layout tab -> click camera -> Track toggle.
   Pink person capsules appear in the 3D viewport.

4. Start playback - Click Bake, then Start. The 40 Hz DMX loop begins.

5. Walk on stage - Within 2 seconds, heads light up and aim at you.
   As you move, beams follow in real-time.

### Comportement multi-personnes

| Personnes visibles | Avec 2 lyres |
| --- | --- |
| 1 person | Both heads aim at the same person |
| 2 people | One head per person (auto-spread) |
| 3+ people | Heads cycle through people every 2s |

# 17. Référence rapide de l'API

All endpoints are served from the Orchestrator at http://localhost:8080.

## Fixtures

| Method | Endpoint | Description |
| --- | --- | --- |
| GET/POST | /api/fixtures | List / create |
| GET/PUT/DELETE | /api/fixtures/:id | CRUD |
| PUT | /api/fixtures/:id/aim | Set DMX aim point |
| POST | /api/fixtures/:id/resolve | Compute pixel positions |

## DMX Profiles

| Method | Endpoint | Description |
| --- | --- | --- |
| GET/POST | /api/dmx-profiles | List / create |
| GET/PUT/DELETE | /api/dmx-profiles/:id | CRUD |
| GET | /api/dmx-profiles/export | Export bundle |
| POST | /api/dmx-profiles/import | Import bundle |
| POST | /api/dmx-profiles/ofl/import-json | Import OFL fixture |

## Timelines

| Method | Endpoint | Description |
| --- | --- | --- |
| GET/POST | /api/timelines | List / create |
| POST | /api/timelines/:id/bake | Start baking |
| POST | /api/timelines/:id/start | Start playback |
| POST | /api/timelines/:id/stop | Stop playback |

## DMX Output

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/dmx/status | Engine status |
| POST | /api/dmx/start | Start engine |
| POST | /api/dmx/stop | Stop engine |
| GET | /api/dmx/fixture/:id/channels | Fixture channels with values |

## Calibration

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /api/cameras/:id/aruco/capture | Capture ArUco frame |
| POST | /api/cameras/:id/aruco/compute | Compute intrinsics |
| POST | /api/cameras/:id/stage-map | Compute camera pose |
| POST | /api/calibration/mover/:id/start | Start mover calibration |
| GET | /api/calibration/mover/:id/status | Poll calibration progress |
| POST | /api/calibration/stereo/calibrate | Build stereo engine |
| POST | /api/calibration/stereo/triangulate | Triangulate 3D point |

## CV Engine

| Method | Endpoint | Description |
| --- | --- | --- |
| GET | /api/cv/status | Model loading status |
| POST | /api/cameras/:id/detect | Object detection (local) |
| POST | /api/cameras/:id/depth | Depth estimation (local) |
| POST | /api/cameras/:id/beam-detect | Beam detection (local) |

## Space

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /api/space/scan | Start point cloud scan |
| GET | /api/space | Get point cloud |
| POST | /api/space/analyze | Detect surfaces |
| GET | /api/project/export | Export project (with spatial data) |
| POST | /api/project/import | Import project |